Coordinate Conversion Application

| THIS TABLE IS UNCLASSIFIED IN ITS ENTIREITY |  |  |  |
| --- | --- | --- | --- |
| Test ID: | CCA-1 | Test Name: | Coordinate Conversion Application |
| Test Objective: |  |  |  |
| 1. Verify that individual coordinate conversion executes correctly. 2. Verify that batch coordinate conversion executes correctly. 3. Verify that user defined ellipsoids and datums can be created. 4. Verify that user defined ellipsoids and datums can be deleted. 5. Verify that output format options can be selected. 6. Verify that online help and version information can be accessed. 7. Verify basic interface functionality. |  |  |  |
| Test Input: |  |  |  |
| User input as directed by the test procedures. |  |  |  |
| Requirements: |  |  |  |
| SRD527 Geodetic SRD528 Geocentric SRD529 Local Cartesian SRD530 Albers Equal Area Conic SRD531 Bonne SRD532 Cassini SRD533 Cylindrical Equal Area SRD534 Lambert Conformal Conic, parallel1 SRD535 Lambert Conformal Conic, parallel2 SRD536 Mercator SRD537 Ney's SRD538 Oblique Mercator SRD539 Polar Stereographic SRD540 Polyconic SRD541 Sinusoidal SRD542 Transverse Cylindrical Equal Area SRD543 Transverse Mercator SRD544 Azimuth Equidistant SRD545 Eckert IV SRD546 Eckert VI SRD547 Equidistant Cylindrical SRD548 Gnomonic SRD549 Miller Cylindrical SRD550 Mollweide SRD551 Orthographic SRD552 Stereographic SRD553 Van der Grinten SRD554 World Geographic Reference System SRD555 Universal Polar Stereographic (UPS) SRD556 Universal Transverse Mercator (UTM) SRD557 Military Grid Reference System (MGRS) SRD558 British National Grid (BNG) SRD559 New Zealand Map Grid (NZMG) SRD560 F-16 Grid Reference System SRD561 United States National Grid (USNG) SRD562 Global Area Reference System (GARS) SRD563 Authalic Sphere SRD564 2D to 3D Conversion SRD566 Horizontal Datum SRD567 User Defined Datums SRD568 Three Character Code Datum Descriptor SRD569 Five Character Code Datum Descriptor SRD570 Six Character Code Datum Descriptor SRD571 Vertical Position Component SRD572 Vertical Position Component Geodetic Height SRD573 Vertical Position Component Mean Sea Level (MSL) Height SRD575 User Defined Ellipsoids SRD576 Accuracy Conversion SRD577 Height Conversion SRD579 Geoid Models Model 1996 15 Minute Grid Bilinear Interpolation SRD580 Geoid Models EGM 84 10 Degree Grid Bilinear Interpolation SRD581 Geoid Models EGM 84 10 Degree Grid Natural Spline Interpolation SRD582 Geoid Models Model 1996 15 Minute Grid Natural Spline Interpolation SRD584 Geodetic Units SRD585 Geodetic Units Degrees and Decimal Degrees SRD586 Geodetic Units Degrees, Minutes, and Decimal Minutes SRD589 Geodetic Units Degrees, Minutes, Seconds, and Decimal Seconds SRD590 Geodetic Units DDDMMSSH SRD591 Geodetic Units DDDMMH SRD592 Geodetic Units DDDH SRD593 Hemisphere Designator “+” or “-“ SRD594 Hemisphere Designator N, S, E, W SRD595 Hemisphere Designator “+” or “-“ vs. N, S, E, W SRD597 Location Formats SRD600 Coordinate Conversion Application GUI SRD601 Coordinate Conversion Application GUI Coordinate Systems SRD602 Coordinate Conversion Application GUI Datums SRD603 Batch Mode Input SRD605 Batch Mode Datum Display SRD606 Batch Mode Ellipsoid Display SRD607 Batch Mode Coordinate System Name Display SRD608 Batch Mode Associated Parameters Display SRD610 Batch Mode Output Display Datums SRD611 Batch Mode Output Display Ellipsoid SRD612 Batch Mode Output Display Coordinate Reference Frames SRD613 Batch Mode Output Display Parameter Fields SRD617 Batch Mode Coordinate Accuracy SRD620 Coordinate Direct Input SRD621 Coordinate Input Conversion SRD623 Input Data displays SRD624 Output Data Displays SRD626 Angular Formats in Fractional Degrees SRD627 Angular Formats in Decimal Minutes SRD628 Angular Formats in Decimal Seconds SRD629 Separators SRD630 Hemisphere Designation SRD631 Longitude Range Scheme SRD632 Precision Scheme SRD634 Coordinate Displays Datum SRD635 Coordinate Displays Default Datum SRD636 Coordinate Displays Coordinate Systems SRD637 Coordinate Displays Default Coordinate Systems SRD638 Coordinate Displays Optional Height SRD639 Coordinate Displays Ellipsoid SRD640 Coordinate Displays Accuracy Values Input SRD641 Coordinate Displays Circular Error Input SRD642 Coordinate Displays Linear Error Input SRD643 Coordinate Displays Spherical Error Input SRD645 Coordinate Displays Circular Error output SRD646 Coordinate Displays Linear Error output SRD647 Coordinate Displays Spherical Error output SRD648 Coordinate Displays Coordinate Systems Parameter Inputs SRD649 Coordinate Displays Coordinate Systems Parameter Outputs SRD650 Coordinate Displays Position Coordinates Inputs SRD651 Coordinate Displays Position Coordinates Outputs SRD653 Coordinate Displays List Item Unknown SRD654 Coordinate Displays List Item user defined (1 meter) SRD655 Coordinate Display List Item GPS Precise Positioning Service (PPS) SRD656 Coordinate Display List Item GPS Standard Positioning Service (SPS) SRD657 Coordinate Display List Item 25K City Map SRD658 Coordinate Display List Item 50K Topographic Line Map SRD659 Coordinate Display List Item 50K Combat Chart SRD660 Coordinate Display List Item 100K TLM SRD661 Coordinate Display List Item 250K Joint Operations Graphic (JOG) SRD662 Coordinate Display List Item 500K Tactical Pilotage Chart SRD663 Coordinate Display List Item 1M Operational Navigation Chart SRD664 Coordinate Display List Item DTED Level 1 SRD665 Coordinate Display List Item DTED Level 2 SRD666 Coordinate Display List Item Digital Feature Analysis Data Level 1 SRD667 Coordinate Display List Item DFAD Level 2 SRD668 Coordinate Display List Item Controlled Image Base SRD669 Coordinate Display List Item Vector Smart Map Level 1 SRD670 Coordinate Display List Item VMAP 100 K Level 2 SRD671 Coordinate Display List Item VMAP 50 K Level SRD672 Spherical Projection Designation SRD673 Add User Defined Datum SRD674 Delete User Defined Datum SRD675 Add User Defined Ellipsoid SRD676 Delete User Defined Ellipsoid SRD677 Coordinate Source SRD680 Input Separator Blank SRD681 Input Separator Slash SRD682 Input Separator Colon SRD684 Output Separator Blank SRD685 Output Separator Slash SRD686 Output Separator Colon SRD689 Longitude Input Range (-180 to 180) SRD690 Longitude Input Range (0 to 360) SRD692 Longitude Output Range (-180 to 180) SRD693 Longitude Output Range (0 to 360) SRD696 Angular Units SRD697 Linear Units SRD699 Angular Units SRD700 Linear Units SRD703 Hemisphere Designations (N, S, E, W) Input SRD704 Hemisphere Designations (+/-) Input SRD706 Hemisphere Designations (N, S, E, W) Output SRD707 Hemisphere Designations (+/-) Output SRD708 Specify Datum SRD710 Vertical Coordinates WGS 84 SRD711 Vertical Coordinates User-Selected Geoid SRD712 Online Help SRD713 Error Reporting |  |  |  |
| Pre-Conditions/Assumptions: |  |  |  |
| Coordinate Conversion Application is running on the given test platform. Test CCA 1-1 Interactive Coordinate Conversion should be run immediately after the Coordinate Conversion Application is run so that default values for the application can be observed. For CCA 1-2 Batch File Coordiante Conversion, the required input test file is assumed to be available in a directory on the given platform to which the tester can navigate using standard browsing conventions. A directory in which to store the output files to which the tester can navigate using standard browsing conventions is assumed to be available on the given platform. The tester will have been informed of both the location of the input and output directories prior to running this test. For CCA 1-3 Define Ellipsoid and Datum, it is assumed the datums and ellipsoids to be defined have not already been created previously to this test. |  |  |  |
| Expected Results: |  |  |  |
| Meet the requirements listed above. |  |  |  |
| Table of Test: |  |  |  |
| CCA 1-1 Basic Interface Verification CCA 1-2 Batch File Coordinate Conversion CCA 1-3 Define an Ellipsoid and a Datum CCA 1-4 Delete a User Defined Ellipsoid and a User Defined Datum CCA 1-5 Select Output Format Options CCA 1-6 Access Online Help and Version Information CCA 1-7 Individual Coordinate Conversion |  |  |  |

| THIS TABLE IS UNCLASSIFIED IN ITS ENTIREITY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Step | Input/Action | Expected Result/Action | Data Recording/Comment | P/F |
| CCA 1-1 Basic Interface Verification |  |  |  |  |
|  | Examine the Datum list fields in the upper and lower halves of the interface. | WGE: World Geodetic System 1984 appears in both Datum List fields. WE: WGS 84 Appears in both Ellipsoid fileds. | SRD566 - MSP shall use WGS 84 as the horizontal datum for common reference. This requirement is verified when an MSP operator demonstrates the MSP capability to use WGS 84 as the horizontal datum for common reference. SRD635 - MSP shall set WGS 84 as the default datum. This requirement is verified when an MSP operator demonstrates that MSP sets WGS 84 as the default datum. SRD708 - MSP shall permit all coordinates to be given in reference to a specified datum, with WGS 84 being the default. This requirement is verified when an MSP operator demonstrates that MSP permits all coordinates to be given in reference to a specified datum, with WGS 84 being the default. SRD639 - MSP shall provide a text box for displaying the selected datum's ellipsoid. This requirement is verified when an MSP operator demonstrates that MSP provides a text box for displaying the selected datum’s ellipsoid. |  |
|  | Scroll through datum list field in the upper half of the interface to insure that the correct datums appear | The following datums appear: WGE: World Geodetic System 1984 WGC: World Geodetic System 1972 EUR-7 EUROPEAN 1950, Mean (7 Param) OGB-7 ORDNANCE GB 1936, Mean (7 Para) ADI-M: ADINDAN, Mean ADI-A: ADINDAN, Ethiopia ADI-B: ADINDAN, Sudan ADI-C: ADINDAN, Mali ADI-D: ADINDAN, Senegal ADI-E: ADINDAN, Burkina Faso ADI-F: ADINDAN, Cameroon AFG: AFGOOYE, Somalia AIA: ANTIGUA ISLAND ASTRO 1943 AIN-A: AIN EL ABD 1970, Bahrain AIN-B: AIN EL ABD 1970, Saudi Arabia AMA: AMERICAN SAMOA 1962 ANO: ANNA 1 ASTRO 1965, Cocos Is. ARF-M: ARC 1950, Mean ARF-A: ARC 1950, Botswana ARF-B: ARC 1950, Lesotho ARF-C: ARC 1950, Malawi ARF-D: ARC 1950, Swaziland ARF-E: ARC 1950, Zaire ARF-F: ARC 1950, Zambia ARF-G: ARC 1950, Zimbabwe ARF-H: ARC 1950, Burundi ARS-M: ARC 1960, Kenya & Tanzania ARS-A: ARC 1960, Kenya ARS-B: ARC 1960, Tanzania ASC: ASCENSION ISLAND 1958 ASM: MONTSERRAT ISLAND ASTRO 1958 ASQ: ASTRO STATION 1952, Marcus Is. ATF: ASTRO BEACON E 1945, Iwo Jima AUA: AUSTRALIAN GEODETIC 1966 AUG: AUSTRALIAN GEODETIC 1984 BAT: DJAKARTA, INDONESIA BID: BISSAU, Guinea-Bissau BER: BERMUDA 1957, Bermuda Islands BOO: BOGOTA OBSERVATORY, Colombia BUR: BUKIT RIMPAH, Banka & Belitung CAC: CAPE CANAVERAL, Fla & Bahamas CAI: CAMPO INCHAUSPE 1969, Arg. CAO: CANTON ASTRO 1966, Phoenix Is. CAP: CAPE, South Africa CAZ: CAMP AREA ASTRO, Camp McMurdo CCD: S-JTSK, Czech Republic CGE: CARTHAGE, Tunisia CHI: CHATHAM ISLAND ASTRO 1971, NZ CHU: CHUA ASTRO, Paraguay COA: CORREGO ALEGRE, Brazil DAL: DABOLA, Guinea DID: DECEPTION ISLAND DOB: GUX 1 ASTRO, Guadalcanal Is. EAS: EASTER ISLAND 1967 ENW: WAKE-ENIWETOK 1960 EST: ESTONIA, 1937 EUR-M: EUROPEAN 1950, Mean (3 Param) EUR-A: EUROPEAN 1950, Western Europe EUR-B: EUROPEAN 1950, Greece EUR-C: EUROPEAN 1950, Norway & Finland EUR-D: EUROPEAN 1950, Portugal & Spain EUR-E: EUROPEAN 1950, Cyprus EUR-F: EUROPEAN 1950, Egypt EUR-G: EUROPEAN 1950, England, Channel EUR-H: EUROPEAN 1950, Iran EUR-I: EUROPEAN 1950, Sardinia(Italy) EUR-J: EUROPEAN 1950, Sicily(Italy) EUR-K: EUROPEAN 1950, England, Ireland EUR-L: EUROPEAN 1950, Malta EUR-S: EUROPEAN 1950, Iraq, Israel EUR-T: EUROPEAN 1950, Tunisia EUS: EUROPEAN 1979 FAH: OMAN FLO: OBSERVATORIO MET. 1939, Flores FOT: FORT THOMAS 1955, Leeward Is. GAA: GAN 1970, Rep. of Maldives GEO: GEODETIC DATUM 1949, NZ GIZ: DOS 1968, Gizo Island GRA: GRACIOSA BASE SW 1948, Azores GUA: GUAM 1963 GSE: GUNUNG SEGARA, Indonesia HEN: HERAT NORTH, Afghanistan HER: HERMANNSKOGEL, old Yugoslavia HIT: PROVISIONAL SOUTH CHILEAN 1963 HJO: HJORSEY 1955, Iceland HKD: HONG KONG 1963 HTN: HU-TZU-SHAN, Taiwan IBE: BELLEVUE (IGN), Efate Is. IDN: INDONESIAN 1974 IND-B: INDIAN, Bangladesh IND-I: INDIAN, India & Nepal IND-P: INDIAN, Pakistan INF-A: INDIAN 1954, Thailand ING-A: INDIAN 1960, Vietnam 16N ING-B: INDIAN 1960, Con Son Island INH-A: INDIAN 1975, Thailand INH-A1: INDIAN 1975, Thailand IRL: IRELAND 1965 ISG: ISTS 061 ASTRO 1968, S Georgia IST: ISTS 073 ASTRO 1969, Diego Garc JOH: JOHNSTON ISLAND 1961 KAN: KANDAWALA, Sri Lanka KEG: KERGUELEN ISLAND 1949 KEA: KERTAU 1948, W Malaysia & Sing. KUS: KUSAIE ASTRO 1951, Caroline Is. LCF: L.C. 5 ASTRO 1961, Cayman Brac LEH: LEIGON, Ghana LIB: LIBERIA 1964 LUZ-A: LUZON, Philippines LUZ-B: LUZON, Mindanao Island MAS: MASSAWA, Ethiopia MER: MERCHICH, Morocco MID: MIDWAY ASTRO 1961, Midway Is. MIK: MAHE 1971, Mahe Is. MIN-A: MINNA, Cameroon MIN-B: MINNA, Nigeria MOD: ROME 1940, Sardinia MPO: M'PORALOKO, Gabon MVS: VITI LEVU 1916, Viti Levu Is. NAH-A: NAHRWAN, Masirah Island (Oman) NAH-B: NAHRWAN, United Arab Emirates NAH-C: NAHRWAN, Saudi Arabia NAP: NAPARIMA, Trinidad & Tobago NAR-A: NORTH AMERICAN 1983, Alaska NAR-B: NORTH AMERICAN 1983, Canada NAR-C: NORTH AMERICAN 1983, CONUS NAR-D: NORTH AMERICAN 1983, Mexico NAR-E: NORTH AMERICAN 1983, Aleutian NAR-H: NORTH AMERICAN 1983, Hawaii NAS-A: NORTH AMERICAN 1927, Eastern US NAS-B: NORTH AMERICAN 1927, Western US NAS-C: NORTH AMERICAN 1927, CONUS NAS-D: NORTH AMERICAN 1927, Alaska NAS-E: NORTH AMERICAN 1927, Canada NAS-F: NORTH AMERICAN 1927, Alberta/BC NAS-G: NORTH AMERICAN 1927, E. Canada NAS-H: NORTH AMERICAN 1927, Man/Ont NAS-I: NORTH AMERICAN 1927, NW Terr. NAS-J: NORTH AMERICAN 1927, Yukon NAS-L: NORTH AMERICAN 1927, Mexico NAS-N: NORTH AMERICAN 1927, C. America NAS-O: NORTH AMERICAN 1927, Canal Zone NAS-P: NORTH AMERICAN 1927, Caribbean NAS-Q: NORTH AMERICAN 1927, Bahamas NAS-R: NORTH AMERICAN 1927, San Salv. NAS-T: NORTH AMERICAN 1927, Cuba NAS-U: NORTH AMERICAN 1927, Greenland NAS-V: NORTH AMERICAN 1927, Aleutian E NAS-W: NORTH AMERICAN 1927, Aleutian W NSD: NORTH SAHARA 1959, Algeria OEG: OLD EGYPTIAN 1907 OGB-M: ORDNANCE GB 1936, Mean (3 Para) OGB-A: ORDNANCE GB 1936, England OGB-B: ORDNANCE GB 1936, Eng., Wales OGB-C: ORDNANCE GB 1936, Scotland OGB-D: ORDNANCE GB 1936, Wales OHA-M: OLD HAWAIIAN (CC), Mean OHA-A: OLD HAWAIIAN (CC), Hawaii OHA-B: OLD HAWAIIAN (CC), Kauai OHA-C: OLD HAWAIIAN (CC), Maui OHA-D: OLD HAWAIIAN (CC), Oahu OHI-M: OLD HAWAIIAN (IN), Mean OHI-A: OLD HAWAIIAN (IN), Hawaii OHI-B: OLD HAWAIIAN (IN), Kauai OHI-C: OLD HAWAIIAN (IN), Maui OHI-D: OLD HAWAIIAN (IN), Oahu PHA: AYABELLE LIGHTHOUSE, Djibouti PIT: PITCAIRN ASTRO 1967 PLN: PICO DE LAS NIEVES, Canary Is. POS: PORTO SANTO 1936, Madeira Is. PRP-A: PROV. S AMERICAN 1956, Bolivia PRP-B: PROV. S AMERICAN 1956, N Chile PRP-C: PROV. S AMERICAN 1956, S Chile PRP-D: PROV. S AMERICAN 1956, Colombia PRP-E: PROV. S AMERICAN 1956, Ecuador PRP-F: PROV. S AMERICAN 1956, Guyana PRP-G: PROV. S AMERICAN 1956, Peru PRP-H: PROV. S AMERICAN 1956, Venez PRP-M: PROV. S AMERICAN 1956, Mean PTB: POINT 58, Burkina Faso & Niger PTN: POINT NOIRE 1948, Congo PUK: PULKOVO 1942, Russia PUR: PUERTO RICO & Virgin Is. QAT: QATAR NATIONAL QUO: QORNOQ, South Greenland REU: REUNION, Mascarene Is. SAE: SANTO (DOS) 1965 SAO: SAO BRAZ, Santa Maria Is. SAP: SAPPER HILL 1943, E Falkland Is SAN-M: SOUTH AMERICAN 1969, Mean SAN-A: SOUTH AMERICAN 1969, Argentina SAN-B: SOUTH AMERICAN 1969, Bolivia SAN-C: SOUTH AMERICAN 1969, Brazil SAN-D: SOUTH AMERICAN 1969, Chile SAN-E: SOUTH AMERICAN 1969, Colombia SAN-F: SOUTH AMERICAN 1969, Ecuador SAN-G: SOUTH AMERICAN 1969, Guyana SAN-H: SOUTH AMERICAN 1969, Paraguay SAN-I: SOUTH AMERICAN 1969, Peru SAN-J: SOUTH AMERICAN 1969, Baltra SAN-K: SOUTH AMERICAN 1969, Trinidad SAN-L: SOUTH AMERICAN 1969, Venezuela SCK: SCHWARZECK, Namibia SGM: SELVAGEM GRANDE 1938, Salvage Is SHB: ASTRO DOS 71/4, St. Helena Is. SOA: SOUTH ASIA, Singapore SPK-A: S-42 (PULKOVO 1942), Hungary SPK-B: S-42 (PULKOVO 1942), Poland SPK-C: S-42 (PK42) Former Czechoslov. SPK-D: S-42 (PULKOVO 1942), Latvia SPK-E: S-42 (PK 1942), Kazakhstan SPK-F: S-42 (PULKOVO 1942), Albania SPK-G: S-42 (PULKOVO 1942), Romania SRL: SIERRA LEONE 1960 TAN: TANANARIVE OBSERVATORY 1925 TDC: TRISTAN ASTRO 1968 TIL: TIMBALAI 1948, Brunei & E Malay TOY-A: TOKYO, Japan TOY-B: TOKYO, South Korea TOY-B1: TOKYO, South Korea TOY-C: TOKYO, Okinawa TOY-M: TOKYO, Mean TRN: ASTRO TERN ISLAND (FRIG) 1961 VOI: VOIROL 1874, Algeria VOR: VOIROL 1960, Algeria WAK: WAKE ISLAND ASTRO 1952 YAC: YACARE, Uruguay ZAN: ZANDERIJ, Suriname KGS: KOREAN GEO DATUM 1995, S Korea SIR: SIRGAS, South America | SRD602 - The Coordinate Conversion Capability shall provide the capability to access the unique codes and names of the datums supported. This requirement is verified when an MSP operator displays a drop-down list of datums. SRD634 - MSP shall provide a drop-down list for selecting/displaying a datum. This requirement is verified when an MSP operator demonstrates that MSP provides provide a drop-down list for selecting/displaying a datum in the coordinate data display. |  |
|  | Examine the coordinate list fields in the upper and lower halves of the interface. | Geodetic appears in one field and Universal Transverse Mercator (UTM) appears in the other. | SRD637 shall set the default coordinate system to be Geodetic in one display and UTM in the other. This requirement is verified when an MSP operator demonstrates that MSP sets the default coordinate system to be Geodetic in one display and UTM in the other. |  |
|  | Scroll through the coordinate list field in the upper window to insure that the correct coordinate systems appear | The following coordinate systems appear (those marked with “(S)” are spherical-based coordinated systems): Geodetic GEOREF Global Area Reference System (GARS) Geocentric Local Cartesian Military Grid Reference System (MGRS) United States National Grid (USNG) Universal Transverse Mercator (UTM) Universal Polar Stereographic (UPS) Albers Equal Area Conic Azimuthal Equidistant (S) British National Grid (BNG) Bonne Cassini Cylindrical Equal Area Eckert IV (S) Eckert VI (S) Equidistant Cylindrical (S) Gnomonic (S) Lambert Conformal Conic (1 parallel) Lambert Conformal Conic (2 parallel) Mercator Miller Cylindrical (S) Mollweide (S) Ney’s (Modified Lambert Conformal Conic) New Zealand Map Grid Oblique Mercator Orthographic (S) Polar Stereographic Polyconic Sinusoidal Stereographic (S) Transverse Cylindrical Equal Area Transverse Mercator Van der Grinten F-16 Grid Reference System | SRD563 - MSP shall convert the reference ellipsoid to an authalic sphere for sphere-based projections met. This requirement is verified when an MSP operator demonstrates the MSP capability to convert the reference ellipsoid to an authalic sphere for sphere-based projections. SRD601 - (U) The Coordinate Conversion Capability shall provide the capability to access the unique codes and names of the coordinate system types supported. This requirement is verified when an MSP operator displays the coordinates system drop down list. SRD672 - MSP shall designate each spherical-based coordinate system with an “(S)” after the projection name in the coordinate system drop down display list met. This requirement is verified when an MSP operator demonstrates that the MSP drop-down list for selecting/displaying coordinate systems includes an "(S)" after the projection name for spherical-based coordinate systems as identified in Table 3.2.1.7-1. |  |
|  | Examine the interface to insure there is a text box displaying each datum’s ellipsoid. | WE: WGS 84 appears in a text box for the datums in the upper and lower halves of the interface. | SRD639 - MSP shall provide a text box for displaying the selected datum’s ellipsoid. This requirement is verified when an MSP operator demonstrates that MSP provides a text box for displaying the selected datum’s ellipsoid. |  |
|  | Examine the interface to insure there are text boxes for the input of the following coordinate accuracy values: circular error, linear error, spherical error. | Text boxes labeled: 90% CE 90% LE 90% SE Should appear in the upper and lower halves of the interface. | SRD640 - MSP shall provide text boxes for input entry of three types of coordinate accuracy values. This requirement is verified when an MSP operator demonstrates that MSP provides text boxes for input entry of three types of coordinate accuracy values. SRD641 - MSP shall provide 90% circular error. This requirement is verified when an MSP operator demonstrates that MSP provides a text box for input entry of 90% circular error coordinate accuracy value. SRD642 - MSP shall provide 90% linear error. This requirement is verified when an MSP operator demonstrates that MSP provides a text box for input entry of 90% circular error coordinate accuracy value. SRD643 - MSP shall provide 90% spherical error. This requirement is verified when an MSP operator demonstrates that MSP provides a text box for input entry of 90% spherical error coordinate accuracy value. SRD645 - MSP shall provide 90% circular error. This requirement is verified when an MSP operator demonstrates that MSP provides a text box for output display of 90% circular error coordinate accuracy value. SRD646 - MSP shall provide 90% linear error. This requirement is verified when an MSP operator demonstrates that MSP provides a text box for output display of 90% circular error coordinate accuracy value. SRD647 - MSP shall provide 90% spherical error. This requirement is verified when an MSP operator demonstrates that MSP provides a text box for output display of 90% spherical error coordinate accuracy value. |  |
|  | Scroll through the Sources list field in each half of the interface to insure that all the coordinate sources appear. | The following should appear in the list field: Unknown User defined (1 meter) GPS PPS GPS SPS 1:25,000 City Map 1:50,000 Topographic Line Map (TLM) 1:50,000 Combat Chart 1:100,000 Topographic Line Map (TLM) 1:250,000 Joint Operations Graphic (JOG) 1:500,000 Tactical Pilotage Chart (TPC) 1:1,000,000 Operational Navigation Chart (ONC) Digital Terrain Elevation Data (DTED) Level 1 Digital Terrain Elevation Data (DTED) Level 2 Digital Feature Analysis Data (DFAD) Level 1 Digital Feature Analysis Data (DFAD) Level 2 Controlled Image Base (CIB) (>10m GSD) Vector Smart Map (VMAP) Level 1 Vector Smart Map (VMAP) Level 2 (1:100,000) Vector Smart Map (VMAP) Level 2 (1:50,000) | SRD653 - MSP shall allow “Unknown” This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: “Unknown”. SRD654 - MSP shall allow "User defined (1 meter)" This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: "User defined (1 meter)". SRD655 - MSP shall allow GPS PPS This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: GPS PPS. SRD656 - MSP shall allow GPS SPS This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: GPS SPS SRD657 - MSP shall allow 1:25,000 City Map This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: 1:25,000 City Map. SRD658 - MSP shall allow 1:50,000 Topographic Line Map (TLM) This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: 1:50,000 Topographic Line Map (TLM). SRD659 - MSP shall allow 1:50,000 Combat Chart This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: 1:50,000 Combat Chart SRD660 - MSP shall allow 1:100,000 TLM This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: 1:100,000 TLM. SRD661 - MSP shall allow 1:250,000 Joint Operations Graphic (JOG) This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: 1:250,000 Joint Operations Graphic (JOG). SRD662 - MSP shall allow 1:500,000 Tactical Pilotage Chart (TPC) This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: 1:500,000 Tactical Pilotage Chart (TPC). SRD663 - MSP shall allow 1:1,000,000 Operational Navigation Chart (ONC) This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: 1:1,000,000 Operational Navigation Chart (ONC). SRD664 - MSP shall allow DTED Level 1 This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: DTED Level 1. SRD665 - MSP shall allow DTED Level 2 This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: DTED Level 2. SRD666 - MSP shall allow Digital Feature Analysis Data (DFAD) Level 1 This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: Digital Feature Analysis Data (DFAD) Level 1. SRD667 - MSP shall allow DFAD Level 2 This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: DFAD Level 2. SRD668 - MSP shall allow Controlled Image Base (CIB) (>10m GSD) This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: Controlled Image Base (CIB) (>10m GSD). SRD669 - MSP shall allow Vector Smart Map (VMAP) Level 1 This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: Vector Smart Map (VMAP) Level 1. SRD670 - MSP shall allow VMAP Level 2 (1:100,000) This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: VMAP Level 2 (1:100,000). SRD671 - MSP shall allow VMAP Level 2 (1:50,000) This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: VMAP Level 2 (1:50,000). |  |
|  | From the coordinate system list field in the lower half of the interface, select: Geodetic | Geodetic appears in the coordinate system field. In addition, a height list field appears in the lower half of the interface set to the value: Ellipsoid Height Also, the height list field in the upper half of the interface becomes active and is set to the value: Ellipsoid Height |  |  |
|  | From the Sources list field in the upper half of the interface, select: Unknown | Unknown appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface all display the same value: Unk | SRD653 - MSP shall allow “Unknown” This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: “Unknown”. |  |
|  | From the Sources list field in the upper half of the interface, select: User defined (1 meter) | User defined (1 meter) appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 1 1 2 | SRD654 - MSP shall allow "User defined (1 meter)" This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: "User defined (1 meter)". |  |
|  | From the Sources list field in the upper half of the interface, select: GPS PPS | GPS PPS appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 20 20 26 | SRD655 - MSP shall allow GPS PPS This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: GPS PPS. |  |
|  | From the Sources list field in the upper half of the interface, select: GPS SPS | GPS SPS appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface all display the same value: 20 20 26 | SRD656 - MSP shall allow GPS SPS This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: GPS SPS |  |
|  | From the Sources list field in the upper half of the interface, select: 1:25,000 City Map | 1:25,000 City Map appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 50 20 52 | SRD657 - MSP shall allow 1:25,000 City Map This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: 1:25,000 City Map. |  |
|  | From the Sources list field in the upper half of the interface, select: 1:50,000 Topographic Line Map (TLM) | 1:50,000 Topographic Line Map (TLM) appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 50 20 52 | SRD658 - MSP shall allow 1:50,000 Topographic Line Map (TLM) This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: 1:50,000 Topographic Line Map (TLM). |  |
|  | From the Sources list field in the upper half of the interface, select: 1:50,000 Combat Chart | 1:50,000 Combat Chart appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 50 20 52 | SRD659 - MSP shall allow 1:50,000 Combat Chart This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: 1:50,000 Combat Chart |  |
|  | From the Sources list field in the upper half of the interface, select: 1:100,000 Topographic Line Map (TLM | 1:100,000 Topographic Line Map (TLM appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 100 20 100 | SRD660 - MSP shall allow 1:100,000 TLM This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: 1:100,000 TLM. |  |
|  | From the Sources list field in the upper half of the interface, select: 1:250,000 Joint Operations Graphic (JOG) | 1:250,000 Joint Operations Graphic (JOG) appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 250 100 258 | SRD661 - MSP shall allow 1:250,000 Joint Operations Graphic (JOG) This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: 1:250,000 Joint Operations Graphic (JOG). |  |
|  | From the Sources list field in the upper half of the interface, select: 1:500,000 Tactical Pilotage Chart (TPC) | 1:500,000 Tactical Pilotage Chart (TPC) appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface all display the same value: 1,000 150 993 | SRD662 - MSP shall allow 1:500,000 Tactical Pilotage Chart (TPC) This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: 1:500,000 Tactical Pilotage Chart (TPC). |  |
|  | From the Sources list field in the upper half of the interface, select: 1:1,000,000 Operational Navigation Chart (ONC) | 1:1,000,000 Operational Navigation Chart (ONC) appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 2,000 650 2,031 | SRD663 - MSP shall allow 1:1,000,000 Operational Navigation Chart (ONC) This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: 1:1,000,000 Operational Navigation Chart (ONC). |  |
|  | From the Sources list field in the upper half of the interface, select: Digital Terrain Elevation Data (DTED ) Level 1 | Digital Terrain Elevation Data (DTED ) Level 1 appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 50 30 55 | SRD664 - MSP shall allow DTED Level 1 This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: DTED Level 1. |  |
|  | From the Sources list field in the upper half of the interface, select: Digital Terrain Elevation Data (DTED ) Level 2 | Digital Terrain Elevation Data (DTED ) Level 2 appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 23 18 27 | SRD665 - MSP shall allow DTED Level 2 This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: DTED Level 2. |  |
|  | From the Sources list field in the upper half of the interface, select: Digital Feature Analysis Data (DFAD) Level 1 | Digital Feature Analysis Data (DFAD) Level 1 appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 130 Unk Unk | SRD666 - MSP shall allow Digital Feature Analysis Data (DFAD) Level 1 This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: Digital Feature Analysis Data (DFAD) Level 1. |  |
|  | From the Sources list field in the upper half of the interface, select: Digital Feature Analysis Data (DFAD) Level 2 | Digital Feature Analysis Data (DFAD) Level 2 appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 130 Unk Unk | SRD667 - MSP shall allow DFAD Level 2 This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: DFAD Level 2. |  |
|  | From the Sources list field in the upper half of the interface, select: Controlled Image Base (CIB) (>10m GSD) | Controlled Image Base (CIB) (>10m GSD) appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 25 Unk Unk | SRD668 - MSP shall allow Controlled Image Base (CIB) (>10m GSD) This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: Controlled Image Base (CIB) (>10m GSD). |  |
|  | From the Sources list field in the upper half of the interface, select: Vector Smart Map (VMAP) Level 1 | Vector Smart Map (VMAP) Level 1 appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 250 100 258 | SRD669 - MSP shall allow Vector Smart Map (VMAP) Level 1 This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: Vector Smart Map (VMAP) Level 1. |  |
|  | From the Sources list field in the upper half of the interface, select: Vector Smart Map (VMAP) Level 2 (1:100,000) | Vector Smart Map (VMAP) Level 2 (1:100,000) appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 100 20 100 | SRD670 - MSP shall allow VMAP Level 2 (1:100,000) This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: VMAP Level 2 (1:100,000). |  |
|  | From the Sources list field in the upper half of the interface, select: Vector Smart Map (VMAP) Level 2 (1:50,000) | Vector Smart Map (VMAP) Level 2 (1:50,000) appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 50 20 52 | SRD671 - MSP shall allow VMAP Level 2 (1:50,000) This requirement is verified when an MSP operator demonstrates that MSP provides in each display a drop-down list for selecting/displaying coordinate sources: VMAP Level 2 (1:50,000). SRD677 - MSP shall automatically populate the coordinate accuracy value text boxes following user selection of a coordinate source. This rqmt is verified when an MSP operator demonstrates that MSP automatically populates the coordinate accuracy value text boxes following operator selection of a coordinate source. |  |
|  | From the Datum list field in the upper half of the interface, select: WGE: World Geodetic System 1984 | WGE: World Geodetic System 1984 appears in the Datum field in the upper half of the screen. |  |  |
|  | From the coordinate system list field in the upper half of the interface, select: Geodetic | Geodetic appears in the coordinate system field. | SRD636 - MSP shall provide a drop-down list for selecting/displaying one of the coordinate systems described in Table 3.2.1.7-1. This requirement is verified when an MSP operator demonstrates that MSP provides a drop-down list for selecting/displaying one of the coordinate systems described in Table 3.2.1.7-1. SRD648 - MSP shall provide text boxes for input entry of coordinate system parameters. This requirement is verified when an MSP operator demonstrates that MSP provides text boxes for input entry of coordinate system parameters. |  |
|  | From the Sources list field in the upper half of the interface, select: Digital Terrain Elevation Data (DTED) Level 1 | Digital Terrain Elevation Data (DTED) Level 1 appears in the Sources field in the upper half of the screen. In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 50 30 55 |  |  |
|  | In the Longitude field in the upper half of the interface, enter: 75 27 25.2W | 75 27 25.2W appears in the Longitude field. |  |  |
|  | In the Latitude field in the upper half of the interface, enter: 43 14 44.5N | 43 14 44.5N appears in the Latitude field. | SRD620 - MSP shall provide direct input of coordinates of individual positions (point-by-point conversions) by means of the coordinate conversion application. This requirement is verified when an MSP operator demonstrates that MSP provides direct input of coordinates of individual positions (point-by-point conversions) by means of the coordinate conversion application. SRD648 - MSP shall provide text boxes for input entry of coordinate system parameters. This requirement is verified when an MSP operator demonstrates that MSP provides text boxes for input entry of coordinate system parameters. |  |
|  | In the elevation field in the upper half of the interface, enter: 0 | 0 appears in the elevation field |  |  |
|  | From the Datum list field in the lower half of the interface, select: WGE: World Geodetic System 1984 | WGE: World Geodetic System 1984 appears in the Datum field in the lower half of the screen. |  |  |
|  | From the coordinate system list field in the lower half of the interface, select: Geocentric | Geocentric appears in the coordinate system field. | SRD649 - MSP shall provide text boxes for output display of coordinate system parameters. This requirement is verified when an MSP operator demonstrates that MSP provides text boxes for output display of coordinate system parameters. |  |
|  | Left click on the Convert: Upper -> Lower button. | The X, Y, And Z fields in the lower half of the interface will respectively display: 1168473 -4504201 4347424 In addition, the circular error (90% CE), linear error (90% LE) and spherical error (90% SE) text boxes in the upper half of the interface display the following values respectively: 50 30 55 | SRD600 - MSP shall provide a GUI for the functionality of the Coordinate Conversion capability. This requirement is verified when an MSP operator demonstrates that MSP provides a GUI for the functionality of the Coordinate Conversion. SRD620 - MSP shall provide direct input of coordinates of individual positions (point-by-point conversions) by means of the coordinate conversion application. This requirement is verified when an MSP operator demonstrates that MSP provides direct input of coordinates of individual positions (point-by-point conversions) by means of the coordinate conversion application. SRD621 - MSP shall provide conversion of coordinates of individual positions (point-by-point conversions) by means of the coordinate conversion application. This requirement is verified when an MSP operator demonstrates that MSP provides conversion of coordinates of individual positions (point-by-point conversions) by means of the coordinate conversion application. This requirement is verified when an MSP operator demonstrates that MSP provides the user the capability to use either of the two coordinate data displays as input or output. SRD623 - MSP shall provide input position coordinates to either display. This requirement is verified when an MSP operator demonstrates that MSP provides input position coordinates to either display. SRD624 - MSP shall provide output of converted coordinates to either display. This requirement is verified when an MSP operator demonstrates that MSP provides the capability to direct the output of converted coordinates to either display. SRD649 - MSP shall provide text boxes for output display of coordinate system parameters. This requirement is verified when an MSP operator demonstrates that MSP provides text boxes for output display of coordinate system parameters. SRD650 - MSP shall provide text boxes for input entry of position coordinates. This requirement is verified when an MSP operator demonstrates that MSP provides text boxes for input entry of coordinates. SRD651 - MSP shall provide text boxes for output display of position coordinates. This requirement is verified when an MSP operator demonstrates that MSP provides text boxes for output display of coordinates. |  |
|  | From the coordinate system list field in the lower half of the interface, select: Geodetic | Geodetic appears in the coordinate system field. |  |  |
|  | In the Height (m) field in the upper half of the interface, enter: 139 | 139 appears in the Height (m) field in the upper half of the interface. |  |  |
|  | In the upper half of the interface, select the following value from the height list field: MSL-EGM96-15M-BL Height (recommended) | MSL-EGM96-15M-BL Height (recommended) appears in the height list field in the upper half of the interface |  |  |
|  | Left click on the Convert: Upper -> Lower button. | 106 appears in the Height (m) field in the lower half of the interface. | SRD579 - MSP shall allow Earth Gravity Model 1996 (EGM 96) with 15 minute by 15 minute grid spacing and bilinear interpolation. This requirement is verified when an MSP operator demonstrates the MSP capability to allow EGM 96 with 15 minute by 15 minute grid spacing and bilinear interpolation. |  |
|  | In the upper half of the interface, select the following value from the height list field: MSL-EGM96-VG-NS Height | MSL-EGM96-VG-NS Height appears in the height list field in the upper half of the interface |  |  |
|  | Left click on the Convert: Upper -> Lower button. | 106 appears in the Height (m) field in the lower half of the interface. | SRD582 - MSP shall allow EGM 96 with variable grid spacing and natural spline interpolation. This requirement is verified when an MSP operator demonstrates the MSP capability to allow EGM 84 with 10 degree by 10 degree grid spacing and natural spline interpolation. |  |
|  | In the upper half of the interface, select the following value from the height list field: MSL-EGM84-10D-BL Height | MSL-EGM84-10D-10D-BL Height appears in the height list field in the upper half of the interface |  |  |
|  | Left click on the Convert: Upper -> Lower button. | 105 appears in the Height (m) field in the lower half of the interface. | SRD580 - MSP shall allow EGM 84 with 10 degree by 10 degree grid spacing and bilinear interpolation This requirement is verified when an MSP operator demonstrates the MSP capability to allow EGM 84 with 10 degree by 10 degree grid spacing and bilinear interpolation |  |
|  | In the upper half of the interface, select the following value from the height list field: MSL-EGM84-10D-NS Height | MSL-EGM84-10D-10D-NS Height appears in the height list field in the upper half of the interface |  |  |
|  | Left click on the Convert: Upper -> Lower button. | 105 appears in the Height (m) field in the lower half of the interface. | SRD577 - MSP shall convert between Geodetic Height (also known as HAE and Ellipsoid Height) and Mean Sea Level (MSL) Height (also known as elevation and Orthometric Height) measured relative to the surface of a model of the geoid. This requirement is verified when an MSP operator demonstrates the MSP capability to convert between Geodetic Height (also known as HAE and Ellipsoid Height) and Mean Sea Level (MSL) Height (also known as elevation and Orthometric Height) measured relative to the surface of a model of the geoid. SRD581 - MSP shall allow EGM 84 with 10 degree by 10 degree grid spacing and natural spline interpolation. This requirement is verified when an MSP operator demonstrates the MSP capability to allow EGM 84 with 10 degree by 10 degree grid spacing and natural spline interpolation. SRD564 - MSP shall assume a position defined on the reference ellipsoid (Height above Ellipsoid (HAE) = 0) when converting from a projection-based coordinate system to a three dimensional coordinate system. This requirement is verified when an MSP operator demonstrates the MSP capability to assume a position defined on the reference ellipsoid (Height above Ellipsoid (HAE) = 0) when converting from a projection-based coordinate system to a three dimensional coordinate system. SRD571 - MSP shall provide the user the capability to specify the vertical position accompanying the coordinate. This requirement is verified when an MSP operator demonstrates the MSP capability to provide the user the capability to specify the vertical position accompanying the coordinate. SRD572 - MSP shall allow Geodetic Height above the WGS 84 ellipsoid (default, as specified in CJCSI 3900.01C) This requirement is verified when an MSP operator demonstrates the MSP capability to allow Geodetic Height above the WGS 84 ellipsoid (default, as specified in CJCSI 3900.01C) SRD573 - MSP shall allow MSL Height with respect to a model of the geoid selected from paragraph 3.2.1.7.10 This requirement is verified when an MSP operator demonstrates the MSP capability to allow MSL Height with respect to a model of the geoid selected from paragraph 3.2.1.7.10 SRD638 - MSP shall, when the Geodetic Coordinate System is active, provide the user the option of specifying whether the height coordinate is ellipsoid height or a specified approximation of MSL/geoid. This requirement is verified when an MSP operator demonstrates that MSP, when the Geodetic Coordinate System is active, provides the user the option of specifying whether the height coordinate is ellipsoid height or a specified approximation of MSL/geoid. SRD710 - MSP shall allow height above WGS 84 ellipsoid (default, per CJCSI 3900.01C) This requirement is verified when an MSP operator demonstrates that MSP provides the capability to enter a vertical coordinate as height above WGS-84 ellipsoid (default, per CJCSI 3900.01C). SRD711 - MSP shall allow elevation above a user-specified geoid selected from those identified in paragraph 3.2.1.7.10 This requirement is verified when an MSP operator demonstrates that MSP provides the capability to enter a vertical coordinate as elevation above a user-specified geoid selected from those identified in para 3.2.1.7.11. |  |
|  | In the Longitude field in the upper half of the interface, enter: -75 27 25.2W | -75 27 25.2W appears in the Longitude field. |  |  |
|  | Left click on the Convert: Upper -> Lower button. | An error message box appears with the following message: Error: Input Geodetic: Invalid Longitude: Invalid longitude value: Longitude out of range |  |  |
|  | Left click on the OK button in the error message box. | The error message box closes. | SRD713 - MSP shall report an error if results are invalid. This requirement is verified when an MSP operator demonstrates the MSP capability to display error messages when results are invalid. |  |

| THIS TABLE IS UNCLASSIFIED IN ITS ENTIREITY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Step |  | Input/Action |  | Expected Result/Action |  | Data/Recording Comment | P/F |
| CCA 1-2 Batch File Coordinate Conversion |  |  |  |  |  |  |  |
|  | Left click on the File item in the MSP GEOTRANS 3.0 menu bar. |  | The File options appear. |  |  |  |  |
|  | Left click the following item: Open |  | A standard Open dialog box appears. |  |  |  |  |
|  | Navigate (if necessary) to the directory where the input test files reside. Select the following file: LCC_14.csv |  | A File Processing window appears. The upper half of the window contains information on the input coordinates to be converted and the lower screen contains information on the output coordinates that will be calculated. For the input coordinates, the following fields should have the following values: Datum: WGE: World Geodetic System 1984 Ellipsoid: WE: WGS 84 Central Meridian: 0 0 0.0E Origin Latitude: 60 0 0.0N Scale Factor: 1.00000 False Easting (m): 0 False Northing (m): 0 In addition, the name of the input coordinate system will be displayed: Lambert Conformal Conic (1 Standard Parallel) Projection For the output coordinates, the following fields should have the following values: Datum: WGE: World Geodetic System 1984 Ellipsoid: WE: WGS 84 In addition there should be a coordinate field containing: Geodetic and a height field containing: No Height |  | SRD605 - Batch Mode Datum Display. This requirement is verified when an MSP operator demonstrates that MSP displays the datum in the selected batch mode input file header. SRD606 - Batch Mode Ellipsoid Display This requirement is verified when an MSP operator demonstrates that MSP displays the ellipsoid in the selected batch mode input file header. SRD607 - Batch Mode Coordinate System Name Display. This requirement is verified when an MSP operator demonstrates that MSP displays the coordinate system name in the selected batch mode input file header SRD608 - Batch Mode Associated Parameters Display. This requirement is verified when an MSP operator demonstrates that MSP displays the associated parameters in the selected batch mode input file header. In this case, this would be: Central Meridian, Origin Latitude, Scale Factor, False Easting, False Northing SRD611 - MSP shall display Ellipsoid field. This requirement is verified when an MSP operator demonstrates the MSP capability to, upon selection of an input file, display an Ellipsoid field. |  |  |
|  | At this point, the input file should be opened and examined in a separate window using a text editor native to the platform on which testing is being conducted. |  | Upon examination, the input file will have the following format: A file header that specifies the datum and the coordinate reference frame type of the input coordinates, including any associated parameter values; followed by sets of input coordinate values, one set per line, with individual values separated by commas followed by at least one space. |  |  |  |  |
|  | From the Datum list field in the lower half of the interface, select: WGE: World Geodetic System 1984 |  | WGE: World Geodetic System 1984 appears in the Datum field in the lower half of the screen. |  | SRD610 - MSP shall display Datum drop-down list. This requirement is verified when an MSP operator demonstrates the MSP capability to, upon selection of an input file, display a Datum drop-down list. |  |  |
|  | From the coordinate system list field in the lower half of the interface, select: Geodetic |  | Geodetic appears in the coordinate system field. In addition, the height field appears with the value: No Height |  | SRD612 - MSP shall display Drop-down list of all coordinate reference frames. This requirement is verified when an MSP operator demonstrates the MSP capability to, upon selection of an input file, display a drop-down list of all coordinate reference frames. SRD613 - MSP shall display Parameter fields. This requirement is verified when an MSP operator demonstrates that MSP displays the parameters for the output file. In this case, just the height field. |  |  |
|  | Left Click on the OK button. |  | A standard Save as… dialog box appears. |  |  |  |  |
|  | Specify where to save the output file containing the converted coordinates. In the File name field, enter: test_output.dat |  | test_output.dat appears in the Filename field. |  |  |  |  |
|  | Left Click on the Save button. |  | The Save as… dialog box closes and a File Processing dialog box appears. |  |  |  |  |
|  | Left Click on the Start button. |  | The coordinate conversion is performed on the coordinates in the input file. When completed, the following will be displayed in the File Processing dialog box: Completed Number of coordinates processed: 300 Number of warnings: 0 Number of errors: 0 Elapsed Time 0.032s (the elapsed time will vary from test to test; the test procedure CCS-3 tests the performance of the CCS software using the elapsed time.) |  | SRD603 - MSP shall provide the capability to convert the coordinates of text file-based input in the following format: A file header that specifies the datum and the coordinate reference frame type of the input coordinates, including any associated parameter values; followed by sets of input coordinate values, one set per line, with individual values separated by commas followed by at least one space This requirement is verified when an MSP operator demonstrates the MSP capability to convert the coordinates of text file-based input in the following format: A file header that specifies the datum and the coordinate reference frame type of the input coordinates, including any associated parameter values; followed by sets of input coordinate values, one set per line, with individual values separated by commas followed by at least one space |  |  |
|  | At this point, the output file should opened, and examined using a text editor native to the platform on which testing is being conducted. |  | The output file will have the same input as the input file: A file header that specifies the datum and the coordinate reference frame type of the output coordinates, including any associated parameter values; followed by sets of output coordinate values, one set per line, with individual values separated by commas followed by at least one space. Although the formats will be the same, actual values will of course differ. |  | SRD615 - This requirement is verified when an MSP operator demonstrates that MSP displays output files identical in format to input files. |  |  |
|  | Click on the OK button. |  | The File Processing Dialog box closes. |  |  |  |  |

| THIS TABLE IS UNCLASSIFIED IN ITS ENTIREITY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Input/Action | Expected Result/Action |  | Data Recording Comment |  | P/F |  |
|  | CCA 1-3 Define an Ellipsoid and a Datum |  |  |  |  |  |  |
|  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  | Under Geodetic Units, select the following radio button: Degrees/Minutes/Seconds |  | The Degrees/Minutes/Seconds radio button is selected. |  |  |  |  |
|  | Under Geodetic Separator, select the following radio button: DD MM SS |  | The DD MM SS radio button is selected. |  |  |  |  |
|  | Under Sign/Hemisphere, select the following radio button: N/S, E/W |  | The N/S, E/W radio button is selected. |  |  |  |  |
|  | Under Longitude Range, select the following radio button: (-180, 180) |  | The (-180, 180) radio button is selected. |  |  |  |  |
|  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  | Under Precision, select the following radio button: 0.01m 0.001 Second |  | The 0.01m 0.001 Second radio button is selected. A message box appears stating: Warning: Output accuracy is limited to ~1m |  |  |  |  |
|  | Left Click on the OK button in the warning box. |  | The warning message box closes. |  |  |  |  |
|  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  | Left click on the Ellipsoid item in the MSP GEOTRANS 3.0 menu bar. |  | The Ellipsoid options appear. |  |  |  |  |
|  | Left click the following item: Create… |  | The Create Ellipsoid dialog box appears. |  |  |  |  |
|  | In the Code field enter: CK |  | CK appears in the Code field. |  |  |  |  |
|  | In the Name field enter: Clarke 1880 (IGN) |  | Clarke 1880 (IGN) appears in the Name field. |  |  |  |  |
|  | In the Semi-Major Axis field enter: 6378249.200 |  | 6378249.200 appears in the Semi-Major Axis field. |  |  |  |  |
|  | In the Inverse Flattening field enter: 293.466020800 |  | 293.466020800 appears in the Inverse Flattening field. |  |  |  |  |
|  | Left Click on the OK button. |  | The Create Ellipsoid dialog box closes. |  |  |  |  |
|  | Left click on the Datum item in the MSP GEOTRANS 3.0 menu bar. |  | The Datum options appear. |  |  |  |  |
|  | Left click the following item: Create… |  | The Create Datum dialog box appears. |  |  |  |  |
|  | If the 3 Parameter Datum tab is not selected, left click on it and select it. |  | The contents of the 3 Parameter Datum tab appears. |  |  |  |  |
|  | In the Code field enter: BEK |  | BEK appears in the Code field. |  |  |  |  |
|  | In the Name field enter: Bekaa Valley Datum 1920 |  | Bekaa Valley Datum 1920 appears in the Name field. |  |  |  |  |
|  | Scroll through the list of ellipsoids in the Ellipsoid field and select the following value: CK: Clarke 1880 (IGN) |  | CK: Clarke 1880 (IGN) appears in the Ellipsoid field. |  |  |  |  |
|  | In the Delta X(m) field, enter the following value: -183 |  | -183 appears in the Delta X(m) field |  |  |  |  |
|  | In the Delta Y(m) field, enter the following value: -15 |  | -15 appears in the Delta Y(m) field |  |  |  |  |
|  | In the Delta Z(m) field, enter the following value: 273 |  | 273 appears in the Delta Z(m) field |  |  |  |  |
|  | In the Std. Error in X(m) field, enter the following value: 3 |  | 3 appears in the Std. Error in X(m) field |  |  |  |  |
|  | In the Std. Error in Y(m) field, enter the following value: 3 |  | 3 appears in the Std. Error in Y(m) field |  |  |  |  |
|  | In the Std. Error in Z(m) field, enter the following value: 3 |  | 3 appears in the Std. Error in Z(m) field |  |  |  |  |
|  | In the Northern Latitude field, enter the following value: 35N |  | 35N appears in the Northern Latitude field. |  |  |  |  |
|  | In the Southern Latitude field, enter the following value: 32N |  | 32N appears in the Southern Latitude field. |  |  |  |  |
|  | In the Western Longitude field, enter the following value: 34E |  | 34E appears in the Western Longitude field. |  |  |  |  |
|  | In the Eastern Longitude field, enter the following value: 37E |  | 37E appears in the Eastern Longitude field. |  |  |  |  |
|  | Left Click on the OK button. |  | The Create Datum dialog box closes. |  |  |  |  |
|  | From the Datum list field in the upper half of the interface, select: WGE: World Geodetic System 1984 |  | WGE: World Geodetic System 1984 appears in the Datum field in the upper half of the screen. |  |  |  |  |
|  | From the coordinate system list field in the upper half of the interface, select: Geodetic |  | Geodetic appears in the coordinate system field. The appropriate parameter field: Height for this coordinate system is displayed |  |  |  |  |
|  | From the height list field in the upper half of the interface, select: No Height |  | No Height appears in the height field. |  |  |  |  |
|  | From the Datum list field in the lower of the interface, select: BEK: Bekaa Valley Datum 1920 |  | BEK: Bekaa Valley Datum 1920 appears in the Datum field in the lower half of the screen. In addition, CK: Clarke 1880 (IGN) appears in the ellipsoid field in the lower half of the interface. |  |  |  |  |
|  | From the coordinate system list field in the lower half of the interface, select: Geodetic |  | Geodetic appears in the coordinate system field. The appropriate parameter field: Height for this coordinate system is displayed |  |  |  |  |
|  | From the height list field in the lower half of the interface, select: No Height |  | No Height appears in the height field. |  |  |  |  |
|  | In the Longitude field in the lower half of the interface, enter: 35 47 16.77940E |  | 35 47 16.77940E appears in the Longitude field. |  |  |  |  |
|  | In the Latitude field in the lower half of the interface, enter: 34 20 50.72110N |  | 34 20 50.72110N appears in the Latitude field. |  |  |  |  |
|  | Left click on the Convert: Lower -> Upper button. |  | The longitude and latitude fields in the upper half of the interface will respectively display: 35 47 20.490E 34 20 50.360N |  | SRD567 - MSP shall provide the user the capability to define datums using the 4 descriptors; datum code, datum name, ellipsoid, datum shift parameters to the WGS 84 ellipsoid. This requirement is verified when an MSP operator demonstrates the MSP capability to provide the user the capability to define datums using the 4 descriptors; datum code, datum name, ellipsoid, datum shift parameters to the WGS 84 ellipsoid. SRD568 - MSP shall allow three character datum code. This requirement is verified when an MSP operator demonstrates the MSP capability to allow three character datum code. SRD575 - MSP shall provide the user the capability to define ellipsoids using the four attributes two letter ellipsoid code, ellipsoid name, and semi-major axis and any inverse flattening value between 250 and 350. This requirement is verified when an MSP operator demonstrates the MSP capability to create define ellipsoids using the four attributes two letter ellipsoid code, ellipsoid name, and semi-major axis and any inverse flattening value between 250 and 350. SRD673 - MSP shall provide the capability to add user-defined datums to the datum selection list. This requirement is verified when an MSP operator demonstrates the MSP capability to add a user-defined datum to the datum selection list. SRD675 - MSP shall provide the capability to add user-defined ellipsoids to the ellipsoid selection list. |  |  |
|  | Left click on the Datum item in the MSP GEOTRANS 3.0 menu bar. |  | The Datum options appear. |  |  |  |  |
|  | Left click the following item: Create… |  | The Create Datum dialog box appears. |  |  |  |  |
|  | If the 3 Parameter Datum tab is not selected, left click on it and select it. |  | The contents of the 3 Parameter Datum tab appears. |  |  |  |  |
|  | In the Code field enter: BEKAA |  | BEKAA appears in the Code field. |  |  |  |  |
|  | In the Name field enter: Bekaa Valley Datum 1920 5 |  | Bekaa Valley Datum 1920 5 appears in the Name field. |  |  |  |  |
|  | Scroll through the list of ellipsoids in the Ellipsoid field and select the following value: CK: Clarke 1880 (IGN) |  | CK: Clarke 1880 (IGN) appears in the Ellipsoid field. |  |  |  |  |
|  | In the Delta X(m) field, enter the following value: -183 |  | -183 appears in the Delta X(m) field |  |  |  |  |
|  | In the Delta Y(m) field, enter the following value: -15 |  | -15 appears in the Delta Y(m) field |  |  |  |  |
|  | In the Delta Z(m) field, enter the following value: 273 |  | 273 appears in the Delta Z(m) field |  |  |  |  |
|  | In the Std. Error in X(m) field, enter the following value: 3 |  | 3 appears in the Std. Error in X(m) field |  |  |  |  |
|  | In the Std. Error in Y(m) field, enter the following value: 3 |  | 3 appears in the Std. Error in Y(m) field |  |  |  |  |
|  | In the Std. Error in Z(m) field, enter the following value: 3 |  | 3 appears in the Std. Error in Z(m) field |  |  |  |  |
|  | In the Northern Latitude field, enter the following value: 35N |  | 35N appears in the Northern Latitude field. |  |  |  |  |
|  | In the Southern Latitude field, enter the following value: 32N |  | 32N appears in the Southern Latitude field. |  |  |  |  |
|  | In the Western Longitude field, enter the following value: 34E |  | 34E appears in the Western Longitude field. |  |  |  |  |
|  | In the Eastern Longitude field, enter the following value: 37E |  | 37E appears in the Eastern Longitude field. |  |  |  |  |
|  | Left Click on the OK button. |  | The Create Datum dialog box closes. |  |  |  |  |
|  | From the Datum list field in the upper half of the interface, select: WGE: World Geodetic System 1984 |  | WGE: World Geodetic System 1984 appears in the Datum field in the upper half of the screen. |  |  |  |  |
|  | From the coordinate system list field in the upper half of the interface, select: Geodetic |  | Geodetic appears in the coordinate system field. Appropriate parameter fields |  |  |  |  |
|  | From the height list field in the lower half of the interface, select: No Height |  | No Height appears in the height field. |  |  |  |  |
|  | From the Datum list field in the lower of the interface, select: BEKAA: Bekaa Valley Datum 1920 5 |  | BEKAA: Bekaa Valley Datum 1920 5 appears in the Datum field in the lower half of the screen. In addition, CK: Clarke 1880 (IGN) appears in the ellipsoid field in the lower half of the interface. |  |  |  |  |
|  | From the coordinate system list field in the lower half of the interface, select: Geodetic |  | Geodetic appears in the coordinate system field. The appropriate parameter field: Height for this coordinate system is displayed |  |  |  |  |
|  | From the height list field in the upper half of the interface, select: No Height |  | No Height appears in the height field. |  |  |  |  |
|  | In the Longitude field in the lower half of the interface, enter: 35 47 16.77940E |  | 35 47 16.77940E appears in the Longitude field. |  |  |  |  |
|  | In the Latitude field in the lower half of the interface, enter: 34 20 50.72110N |  | 34 20 50.72110N appears in the Latitude field. |  |  |  |  |
|  | Left click on the Convert: Lower -> Upper button. |  | The longitude and latitude fields in the upper half of the interface will respectively display: 35 47 20.490E 34 20 50.360N |  | SRD569 - MSP shall allow five character datum code. This requirement is verified when an MSP operator demonstrates the MSP capability to allow five character datum code. |  |  |
|  | Left click on the Datum item in the MSP GEOTRANS 3.0 menu bar. |  | The Datum options appear. |  |  |  |  |
|  | Left click the following item: Create… |  | The Create Datum dialog box appears. |  |  |  |  |
|  | If the 3 Parameter Datum tab is not selected, left click on it and select it. |  | The contents of the 3 Parameter Datum tab appears. |  |  |  |  |
|  | In the Code field enter: BEKAAV |  | BEKAAV appears in the Code field. |  |  |  |  |
|  | In the Name field enter: Bekaa Valley Datum 1920 6 |  | Bekaa Valley Datum 1920 6 appears in the Name field. |  |  |  |  |
|  | Scroll through the list of ellipsoids in the Ellipsoid field and select the following value: CK: Clarke 1880 (IGN) |  | CK: Clarke 1880 (IGN) Appears in the Ellipsoid field. |  |  |  |  |
|  | In the Delta X(m) field, enter the following value: -183 |  | -183 appears in the Delta X(m) field |  |  |  |  |
|  | In the Delta Y(m) field, enter the following value: -15 |  | -15 appears in the Delta Y(m) field |  |  |  |  |
|  | In the Delta Z(m) field, enter the following value: 273 |  | 273 appears in the Delta Z(m) field |  |  |  |  |
|  | In the Std. Error in X(m) field, enter the following value: 3 |  | 3 appears in the Std. Error in X(m) field |  |  |  |  |
|  | In the Std. Error in Y(m) field, enter the following value: 3 |  | 3 appears in the Std. Error in Y(m) field |  |  |  |  |
|  | In the Std. Error in Z(m) field, enter the following value: 3 |  | 3 appears in the Std. Error in Z(m) field |  |  |  |  |
|  | In the Northern Latitude field, enter the following value: 35N |  | 35N appears in the Northern Latitude field. |  |  |  |  |
|  | In the Southern Latitude field, enter the following value: 32N |  | 32N appears in the Southern Latitude field. |  |  |  |  |
|  | In the Western Longitude field, enter the following value: 34E |  | 34E appears in the Western Longitude field. |  |  |  |  |
|  | In the Eastern Longitude field, enter the following value: 37E |  | 37E appears in the Eastern Longitude field. |  |  |  |  |
|  | Left Click on the OK button. |  | The Create Datum dialog box closes. |  |  |  |  |
|  | From the Datum list field in the upper half of the interface, select: WGE: World Geodetic System 1984 |  | WGE: World Geodetic System 1984 appears in the Datum field in the upper half of the screen. |  |  |  |  |
|  | From the coordinate system list field in the upper half of the interface, select: Geodetic |  | Geodetic appears in the coordinate system field. The appropriate parameter field: Ellipsoid Height for this coordinate system is displayed |  |  |  |  |
|  | From the height list field in the upper half of the interface, select: No Height |  | No Height appears in the height field. |  |  |  |  |
|  | From the Datum list field in the lower of the interface, select: BEKAAV: Bekaa Valley Datum 1920 6 |  | BEKAAV: Bekaa Valley Datum 1920 6 appears in the Datum field in the lower half of the screen. In addition, CK: Clarke 1880 (IGN) appears in the ellipsoid field in the lower half of the interface. |  |  |  |  |
|  | From the coordinate system list field in the lower half of the interface, select: Geodetic |  | Geodetic appears in the coordinate system field. The appropriate parameter field: Height for this coordinate system is displayed |  |  |  |  |
|  | From the height list field in the upper half of the interface, select: No Height |  | No Height appears in the height field. |  |  |  |  |
|  | In the Longitude field in the lower half of the interface, enter: 35 47 16.77940E |  | 35 47 16.77940E appears in the Longitude field. |  |  |  |  |
|  | In the Latitude field in the lower half of the interface, enter: 34 20 50.72110N |  | 34 20 50.72110N appears in the Latitude field. |  |  |  |  |
|  | Left click on the Convert: Lower -> Upper button. |  | The longitude and latitude fields in the upper half of the interface will respectively display: 35 47 20.490E 34 20 50.360N |  | SRD570 - MSP shall allow six character datum code. This requirement is verified when an MSP operator demonstrates the MSP capability to allow six character datum code. |  |  |

| THIS TABLE IS UNCLASSIFIED IN ITS ENTIREITY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Input/Action |  | Expected Results/Action |  | Data Recording Comment |  | P/F |
| CCA 1-4 Delete a User Defined Ellipsoid and a User Defined Datum |  |  |  |  |  |  |  |
|  |  | Left click on the Datum item in the MSP GEOTRANS 3.0 menu bar. |  | The Datum options appear. |  |  |  |
|  |  | Left click the following item: Delete… |  | The Delete Datum dialog box appears. |  |  |  |
|  |  | Scroll through the Datum list field and select the following value: BEK: Bekaa Valley Datum 1920 |  | BEK: Bekaa Valley Datum 1920 appears in the Datum list field. |  |  |  |
|  |  | Left Click on the OK button. |  | The Delete Datum dialog box closes. |  |  |  |
|  |  | Left click on the Datum item in the MSP GEOTRANS 3.0 menu bar. |  | The Datum options appear. |  |  |  |
|  |  | Left click the following item: Delete… |  | The Delete Datum dialog box appears. |  |  |  |
|  |  | Scroll through the Datum list field and select the following value: BEKAA: Bekaa Valley Datum 1920 5 |  | BEKAA: Bekaa Valley Datum 1920 5 appears in the Datum list field. |  |  |  |
|  |  | Left Click on the OK button. |  | The Delete Datum dialog box closes. |  |  |  |
|  |  | Left click on the Datum item in the MSP GEOTRANS 3.0 menu bar. |  | The Datum options appear. |  |  |  |
|  |  | Left click the following item: Delete… |  | The Delete Datum dialog box appears. |  |  |  |
|  |  | Scroll through the Datum list field and select the following value: BEKAAV: Bekaa Valley Datum 1920 6 |  | BEKAAV: Bekaa Valley Datum 1920 6 appears in the Datum list field. |  |  |  |
|  |  | Left Click on the OK button. |  | The Delete Datum dialog box closes. |  |  |  |
|  |  | Left click on the Ellipsoid item in the MSP GEOTRANS 3.0 menu bar. |  | The Ellipsoid options appear. |  |  |  |
|  |  | Left click the following item: Delete… |  | The Delete ellipsoid dialog box appears. |  |  |  |
|  |  | Navigate to the directory where the ellips.dat file resides. Open the file with a text editor and search for: CK: Clarke 1880 (IGN) |  | CK: Clarke 1880 (IGN) will not appear in the file. |  | SRD676 - MSP shall provide the capability to remove user-defined ellipsoids from the ellipsoid selection list. This requirement is verified when an MSP operator demonstrates the MSP capability to remove a user-defined ellipsoid from the ellipsoid selection list. |  |
|  |  | Close the ellips.dat file |  | The file closes. |  |  |  |
|  |  | Left Click on the OK button. |  | The Ellipsoid Datum dialog box closes. |  |  |  |
|  |  | Scroll through the Datum list field in the upper half of the MSP GEOTRANS 3.0 interface and search for the following values: BEK: Bekaa Valley Datum 1920 BEKAA: Bekaa Valley Datum 1920 5 BEKAAV: Bekaa Valley Datum 1920 6 |  | The values in question will not be found as they and their associated ellipsoid have been successfully deleted. |  | SRD674 - MSP shall provide the capability to remove user-defined datums from the datum selection list. This requirement is verified when an MSP operator demonstrates the MSP capability to remove a user-defined datum from the datum selection list. |  |
|  |  | Left click on the Datum item in the MSP GEOTRANS 3.0 menu bar. |  | The Datum options appear. |  |  |  |
|  |  | Left click the following item: Delete… |  | The Delete Datum dialog box appears. In the datum list field, the following value appears: WGE: World Geodetic System 1984 |  |  |  |
|  |  | Left click on the OK button in the Delete Datum dialog box. |  | An error message box appears with the following message: Error: Specified code not user defined. |  | SRD713 - MSP shall report an error if results are invalid. This requirement is verified when an MSP operator demonstrates the MSP capability to display error if results are invalid. messages when results are invalid. |  |
|  |  | Left click on the OK button in the error message box. |  | The error message box closes. |  |  |  |
|  |  | Left click on the Cancel button in the Delete Datum dialog box. |  | The Delete Datum dialog box closes. |  |  |  |

| THIS TABLE IS UNCLASSIFIED IN ITS ENTIREITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Input/Action |  | Expected Results/Action |  | Data Recording Comment |  | P/F |  |
| CCA 1-5 Select Output Format Options |  |  |  |  |  |  |  |  |
|  |  | From the Datum list field in the upper half of the interface, select: WGE: World Geodetic System 1984 |  | WGE: World Geodetic System 1984 appears in the Datum field in the upper half of the screen. |  |  |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Geodetic |  | Geodetic appears in the coordinate system field. |  |  |  |  |
|  |  | From the Datum list field in the lower half of the interface, select: WGE: World Geodetic System 1984 |  | WGE: World Geodetic System 1984 appears in the Datum field in the lower half of the screen. |  |  |  |  |
|  |  | From the coordinate system list field in the lower half of the interface, select: Military Grid Reference System (MGRS) |  | Military Grid Reference System appears in the coordinate system field. |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Geodetic Units, select the following radio button: Degrees |  | The Degrees radio button is selected. |  | SRD589 - MSP shall accept the location data in DDDMMSSH data format. This requirement is verified when an MSP operator demonstrates the MSP capability to accept location data in floating point degrees for the following format DDDMMSSH. SRD626 - MSP shall provide degrees and decimal (fractional) degrees. This requirement is verified when an MSP operator demonstrates that MSP provides degrees and decimal (fractional) degrees as an angular unit input/output format. SRD627 - MSP shall provide degrees, minutes, and decimal minutes. This requirement is verified when an MSP operator demonstrates that MSP provides degrees, minutes, and decimal minutes as an angular unit input/output format. SRD628 - MSP shall provide degrees, minutes, seconds, and decimal seconds. This requirement is verified when an MSP operator demonstrates that MSP provides degrees, minutes, seconds, and decimal seconds as an angular unit input/output format. |  |  |
|  |  | Under Geodetic Separator, select the following radio button: DD MM SS |  | The DD MM SS radio button is selected. |  |  |  |  |
|  |  | Under Sign/Hemisphere, select the following radio button: N/S, E/W |  | The N/S, E/W radio button is selected. |  |  |  |  |
|  |  | Under Longitude Range, select the following radio button: (-180, 180) |  | The (-180, 180) radio button is selected. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 1m 0.1 Second |  | The 1m 0.1 Second radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the interface, enter: 75.454530W |  | 75.454530W appears in the Longitude field. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the interface, enter: 43.245700N |  | 43.245700N appears in the Latitude field. |  | SRD592 - MSP shall accept at least six ASCII characters to the right of the decimal place for location inputs consisting of decimal degrees. This requirement is verified when an MSP operator demonstrates the MSP capability to accept at least six ASCII characters to the right of the decimal place for location inputs consisting of decimal degrees. |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | 18TVN6310088200 appears in the Coordinate String Field in the lower half of the interface. |  | SRD584 - MSP shall allow degrees and decimal (fractional) degrees This requirement is verified when an MSP operator demonstrates host application-specified precision for formatting the following scheme of geodetic units: degrees and decimal (fractional) degrees. SRD591 - MSP shall accept the location data in DDDH data format. This requirement is verified when an MSP operator demonstrates the MSP capability to accept location data in floating point degrees for the following format DDDH. SRD632 - MSP shall include a method for choosing between degrees of precision for the input/output of angular and linear units. This requirement is verified when an MSP operator demonstrates that MSP provides a method for choosing between degrees of precision for the input/output of angular and linear units. |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Geodetic Units, select the following radio button: Degrees/Minutes |  | The Degrees/Minutes radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the interface, enter: 75 27.272W |  | 75 27.272W appears in the Longitude field. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the interface, enter: 43 14.742N |  | 43 14.742N appears in the Latitude field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | 18TVN6310088200 appears in the Coordinate String Field in the lower half of the interface. |  | SRD585 - MSP shall allow degrees, minutes, and decimal minutes This requirement is verified when an MSP operator demonstrates host application-specified precision for formatting the following scheme of geodetic units: degrees, minutes, and decimal minutes. SRD590 - MSP shall accept the location data in DDDMMH data format. This requirement is verified when an MSP operator demonstrates the MSP capability to accept location data in floating point degrees for the following format DDDMMH. |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Geodetic Units, select the following radio button: Degrees/Minutes/Seconds |  | The Degrees/Minutes/Seconds radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the interface, enter: 75 27 16.3W |  | 75 27 16.3W appears in the Longitude field. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the interface, enter: 43 14 44.5N |  | 43 14 44.5N appears in the Latitude field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | 18TVN6310088200 appears in the Coordinate String Field in the lower half of the interface. |  | SRD586 - MSP shall allow degrees, minutes, seconds, and decimal seconds This requirement is verified when an MSP operator demonstrates host application-specified precision for formatting the following scheme of geodetic units: degrees, minutes, seconds, and decimal seconds. SRD589 - MSP shall accept the location data in DDDMMSSH data format. This requirement is verified when an MSP operator demonstrates the MSP capability to accept location data in floating point degrees for the following format DDDMMSSH. SRD597 - MSP shall convert between the location data formats identified in requirement 3.2.1.7.12.1 "ASCII Location Data Formats"(U) This requirement is verified when an MSP operator demonstrates the MSP capability to convert between the location data formats identified in paragraph 3.2.1.7.12.1. |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Geodetic Separator, select the following radio button: DD/MM/SS |  | The DD/MM/SS radio button is selected. |  | SRD629 - MSP shall include a method for choosing a separator between degrees, minutes, and seconds. This requirement is verified when an MSP operator demonstrates that MSP provides a method for choosing a separator. |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | Left click the Convert: Lower -> Upper button. |  | In the upper half of the inter face, the Longitude field displays the following value: 75/27/16.3W and the Latitude field displays the following value: 43/14/44.5N |  | SRD685 - MSP shall allow slash (“/”). This requirement is verified when an MSP operator demonstrates the successful output of geographic coordinates from MSP with the following character serving as a separator between degrees, minutes, and seconds: Slash (“/”). |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Geodetic Separator, select the following radio button: DD:MM:SS |  | The DD:MM:SS radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | Left click the Convert: Lower -> Upper button. |  | In the upper half of the inter face, the Longitude field displays the following value: 75:27:16.3W and the Latitude field displays the following value: 43:14:44.5N |  | SRD686 - MSP shall allow colon (“:”). This requirement is verified when an MSP operator demonstrates the successful output of geographic coordinates from MSP with the following character serving as a separator between degrees, minutes, and seconds: Colon (“:”). |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Geodetic Separator, select the following radio button: DD MM SS |  | The DD MM SS radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | Left click the Convert: Lower -> Upper button. |  | In the upper half of the inter face, the Longitude field displays the following value: 75 27 16.3W and the Latitude field displays the following value: 43 14 44.5N |  | SRD684 - MSP shall allow blank space (“ “). This requirement is verified when an MSP operator demonstrates the successful output of geographic coordinates from MSP with the following character serving as a separator between degrees, minutes, and seconds: Blank space (“ “). |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Geodetic Separator, select the following radio button: DD/MM/SS |  | The DD/MM/SS radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter the value: 75/27/16.3W |  | 75/27/16.3W Appears in the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter the value: 43/14/44.5N |  | 43/14/44.5N appears in the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Upper -> Lower button. |  | In the lower half of the inter face, the Coordinate String field displays the following value: 18TVN6310088200 |  | SRD681 - MSP shall allow slash (“/”). This requirement is verified when an MSP operator demonstrates the successful input of geographic coordinates to MSP with the following character serving as a separator between degrees, minutes, and seconds: Slash (“/”). |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Geodetic Separator, select the following radio button: DD:MM:SS |  | The DD:MM:SS radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter the value: 75:27:16.3W |  | 75:27:16.3W appears in the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter the value: 43:14:44.5N |  | 43:14:44.5N appears in the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Upper -> Lower button. |  | In the lower half of the inter face, the Coordinate String field displays the following value: 18TVN6310088200 |  | SRD682 - MSP shall allow colon (“:”). This requirement is verified when an MSP operator demonstrates the successful input of geographic coordinates to MSP with the following character serving as a separator between degrees, minutes, and seconds: Colon (“:”). |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Geodetic Separator, select the following radio button: DD MM SS |  | The DD MM SS radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter the value: 75 27 16.3W |  | 75 27 16.3W appears in the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter the value: 43 14 44.5N |  | 43 14 44.5N appears in the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Upper -> Lower button. |  | In the lower half of the inter face, the Coordinate String field displays the following value: 18TVN6310088200 |  | SRD680 - MSP shall allow blank space (“ “). This requirement is verified when an MSP operator demonstrates the successful input of geographic coordinates to MSP with the following character serving as a separator between degrees, minutes, and seconds: Blank space (“ “). |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Sign/Hemisphere, select the following radio button: + / - |  | The + / - radio button is selected. |  | SRD594 - MSP shall accept hemisphere designation information as ASCII N, S, E, W (North, South, East, West). This requirement is verified when an MSP operator demonstrates the MSP capability to accept hemispheric information as ASCII N, S, E, W (North, South, East, West) SRD630 - MSP shall include a method for choosing between the two conventions for designating hemispheres (+/- or N, S, E, W). This requirement is verified when an MSP operator demonstrates that MSP provides a method for choosing a separator between degrees, minutes, and seconds. |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | Left click the Convert: Lower -> Upper button. |  | In the upper half of the inter face, the Longitude field displays the following value: -75 27 16.3 and the Latitude field displays the following value: 43 14 44.5 |  | SRD595 - MSP shall accept hemisphere designation information as ASCII "+" and "-", where "+" is equivalent to North of the equator and East of the Greenwich meridian and "-" is equivalent to South and West. This requirement is verified when an MSP operator demonstrates the MSP capability to accept hemispheric information as “+” or “-“ SRD707 - MSP shall allow the +/- convention, where “+” signifies latitude north of the equator and longitude east of the Greenwich meridian to 180 degrees and “–”signifies latitude south of the equator and longitude west of the Greenwich meridian to 180 degrees. This requirement is verified when an MSP operator demonstrates that MSP allows the output of hemisphere by a +/- convention. |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Sign/Hemisphere, select the following radio button: N/S, E/W |  | The N/S, E/W radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | Left click the Convert: Lower -> Upper button. |  | In the upper half of the inter face, the Longitude field displays the following value: 75 27 16.3W and the Latitude field displays the following value: 43 14 44.5N |  | SRD706 - MSP shall allow a character (N, S, E, W) convention, where N signifies north of the equator, S signifies south of the equator, E signifies east of the Greenwich meridian to 180 degrees, and W signifies west of the Greenwich meridian to 180 degrees. This requirement is verified when an MSP operator demonstrates that MSP allows the output of hemisphere by a character (N, S, E, W) convention. |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Sign/Hemisphere, select the following radio button: +/- |  | The +/- radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter the value: -75 27 16.3 |  | -75 27 16.3 appears in the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter the value: 43 14 44.5 |  | 43 14 44.5 appears in the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Upper -> Lower button. |  | In the Lower half of the interface, the Coordinate String field displays the following value: 18TVN6310088200 |  | SRD704 - MSP shall allow the +/- convention, where “+” signifies latitude north of the equator and longitude east of the Greenwich meridian to 180 degrees and “–”signifies latitude south of the equator and longitude west of the Greenwich meridian to 180 degrees. This requirement is verified when an MSP operator demonstrates that MSP allows the input of hemisphere by a +/- convention. |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Sign/Hemisphere, select the following radio button: N/S, E/W |  | The N/S, E/W radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter the value: 75 27 16.3W |  | 75 27 16.3W appears in the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter the value: 43 14 44.5N |  | 43 14 44.5N appears in the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Upper -> Lower button. |  | In the Lower half of the interface, the Coordinate String field displays the following value: 18TVN6310088200 |  | SRD703 - MSP shall allow a character (N, S, E, W) convention, where N signifies north of the equator, S signifies south of the equator, E signifies east of the Greenwich meridian to 180 degrees, and W signifies west of the Greenwich meridian to 180 degrees. This requirement is verified when an MSP operator demonstrates that MSP allows the input of hemisphere by a character (N, S, E, W) convention. |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Longitude Range, select the following radio button: (0, 360) |  | The (0, 360) radio button is selected. |  | SRD631 - MSP shall include a method for choosing between longitude range schemes (+/- 180 or 0-360). This requirement is verified when an MSP operator demonstrates that MSP provides a method for choosing between longitude range schemes (+/- 180 or 0-360). |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | Left click the Convert: Lower -> Upper button. |  | In the upper half of the inter face, the Longitude field displays the following value: 284 32 43.7E |  | SRD693 - MSP shall allow longitude values ranging from 0 to 360. This requirement is verified when an MSP operator demonstrates the successful output of longitude values from MSP using the following schema: Longitude values ranging from 0 to 360. |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Longitude Range, select the following radio button: (-180, 180) |  | The (-180, 180) radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | Left click the Convert: Lower -> Upper button. |  | In the upper half of the inter face, the Longitude field displays the following value: 75 27 16.3W |  | SRD692 - MSP shall allow longitude values ranging from –180 to +180 This requirement is verified when an MSP operator demonstrates the successful output of longitude values from MSP using the following schema: Longitude values ranging from –180 to +180. |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Longitude Range, select the following radio button: (0, 360) |  | The (0, 360) radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter the value: 284 32 43.7E |  | 284 32 43.7E appears in the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  | SRD690 - MSP shall allow longitude values ranging from 0 to 360. This requirement is verified when an MSP operator demonstrates the successful input of longitude values into MSP using the following schema: Longitude values ranging from 0 to 360. |  |  |
|  |  | Left click the Convert: Upper -> Lower button. |  | In the Lower half of the interface, the Coordinate String field displays the following value: 18TVN6310088200 |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Longitude Range, select the following radio button: (-180, 180) |  | The (-180, 180) radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter the value: 75 27 16.3W |  | 75 27 16.3W appears in the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Upper -> Lower button. |  | In the Lower half of the interface, the Coordinate String field displays the following value: 18TVN6310088200 |  | SRD689 - MSP shall allow longitude values ranging from –180 to +180 This requirement is verified when an MSP operator demonstrates the successful input of longitude values into MSP using the following schema: Longitude values ranging from –180 to +180. |  |  |
|  |  | From the Datum list field in the upper half of the interface, select: WGE: World Geodetic System 1984 |  | WGE: World Geodetic System 1984 appears in the Datum field in the upper half of the screen. |  |  |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Geodetic |  | Geodetic appears in the coordinate system field. |  |  |  |  |
|  |  | From the Datum list field in the lower half of the interface, select: WGE: World Geodetic System 1984 |  | WGE: World Geodetic System 1984 appears in the Datum field in the lower half of the screen. |  |  |  |  |
|  |  | From the coordinate system list field in the lower half of the interface, select: Universal Transverse Mercator (UTM) |  | Universal Transverse Mercator (UTM) appears in the coordinate system field. |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Geodetic Units, select the following radio button: Degrees/Minutes/Seconds |  | The Degrees/Minutes/Seconds radio button is selected. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 100,000m 1 Degree |  | The 100,000m 1 Degree Second radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 75W |  | 75W appears in the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 43N |  | 43N appears in the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Upper -> Lower button. |  | In the lower half of the interface, the Easting/X (m) field displays following value: 500000 and the Northing/Y (m) field displays the following value: 4800000 |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 10,000m 10 minutes |  | The 10,000m 10 Minutes radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 75 30W |  | 75 30W appears in the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 43 20N |  | 43 20N appears in the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Upper -> Lower button. |  | In the lower half of the interface, the Easting/X (m) field displays following value: 460000 and the Northing/Y (m) field displays the following value: 4800000 |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 1,000m 1 Minute |  | The 1,000m 1 Minute radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 75 27W |  | 75 27W appears in the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 43 15N |  | 43 15N appears in the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Upper -> Lower button. |  | In the lower half of the interface, the Easting/X (m) field displays following value: 463000 and the Northing/Y (m) field displays the following value: 4789000 |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 100m 10 Seconds |  | The 100m 10 Seconds radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 75 27 20W |  | 75 27 20W appears in the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 43 14 40N |  | 43 14 40N appears in the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Upper -> Lower button. |  | In the lower half of the interface, the Easting/X (m) field displays following value: 463000 and the Northing/Y (m) field displays the following value: 4788100 |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 10m 1 Second |  | The 10m 1 Second radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 75 27 16W |  | 75 27 16W appears in the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 43 14 45N |  | 43 14 45N appears in the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Upper -> Lower button. |  | In the lower half of the interface, the Easting/X (m) field displays following value: 463110 and the Northing/Y (m) field displays the following value: 4788220 |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 1m 0.1 Second |  | The 1m 0.1 Second radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 75 27 16.3W |  | 75 27 16.3W appears in the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 43 14 44.5N |  | 43 14 44.5N appears in the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Upper -> Lower button. |  | In the lower half of the interface, the Easting/X (m) field displays following value: 463100 and the Northing/Y (m) field displays the following value: 4788200 |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 0.1m 0.01 Second |  | The 0.1m 0.01 Second radio button is selected. A message box appears stating: Warning: Output accuracy is limited to ~1M |  |  |  |  |
|  |  | Left Click on the OK button in the warning box. |  | The warning message box closes. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 75 27 16.27W |  | 75 27 16.27W appears in the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 43 14 44.49N |  | 43 14 44.49N appears in the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Upper -> Lower button. |  | In the lower half of the interface, the Easting/X (m) field displays following value: 463100.8 and the Northing/Y (m) field displays the following value: 4788199.3 |  | SRD593 - MSP shall accept at least two ASCII characters to the right of the decimal place for inputs consisting of only seconds. This requirement is verified when an MSP operator demonstrates the MSP capability to accept at least two ASCII characters to the right of the decimal place for inputs consisting of only seconds. |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 0.01m 0.001 Second |  | The 0.01m 0.001 Second radio button is selected. A message box appears stating: Warning: Output accuracy is limited to ~1M |  |  |  |  |
|  |  | Left Click on the OK button in the warning box. |  | The warning message box closes. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 75 27 16.276W |  | 75 27 16.276W appears in the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 43 14 44.488N |  | 43 14 44.488N appears in the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Upper -> Lower button. |  | In the lower half of the interface, the Easting/X (m) field displays following value: 463100.66 and the Northing/Y (m) field displays the following value: 4788199.24 |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 0.001m 0.0001 Second |  | The 0.001m 0.0001 Second radio button is selected. A message box appears stating: Warning: Output accuracy is limited to ~1M |  |  |  |  |
|  |  | Left Click on the OK button in the warning box. |  | The warning message box closes. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 75 27 16.2759W |  | 75 27 16.2759W appears in the Longitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface, enter: 43 14 44.4880N |  | 43 14 44.4880N appears in the Latitude field in the upper half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Upper -> Lower button. |  | In the lower half of the interface, the Easting/X (m) field displays following value: 463100.664 and the Northing/Y (m) field displays the following value: 4788199.240 |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 100,000 m 1 Degree |  | The 100,000 m 1 Degree radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 500000 |  | 500000 appears in the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 4800000 |  | 4800000 appears in the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Lower -> Upper button. |  | In the upper half of the interface, the Longitude field displays following value: 75W and the Latitude field displays the following value: 43N |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 10,000 m 10 Minutes |  | The 10,000 m 10 Minutes radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 460000 |  | 460000 appears in the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 4800000 |  | 4800000 appears in the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Lower -> Upper button. |  | In the upper half of the interface, the Longitude field displays following value: 75 30W and the Latitude field displays the following value: 43 20N |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 1,000 m 1 Minute |  | The 1,000 m 1 Minute radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 463000 |  | 463000 appears in the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 4789000 |  | 4789000 appears in the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Lower -> Upper button. |  | In the upper half of the interface, the Longitude field displays following value: 75 27W and the Latitude field displays the following value: 43 15N |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 100m 10 Seconds |  | The 100m 10 Seconds radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 463000 |  | 463000 appears in the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 4788100 |  | 4788100 appears in the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Lower -> Upper button. |  | In the upper half of the interface, the Longitude field displays following value: 75 27 20W and the Latitude field displays the following value: 43 14 40N |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 10m 1 Second |  | The 10m 1 Second radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 463100 |  | 463100 qppears in the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 4788200 |  | 4788200 appears in the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Lower -> Upper button. |  | In the upper half of the interface, the Longitude field displays following value: 75 27 16W and the Latitude field displays the following value: 43 14 45N |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 1m 0.1 Second |  | The 1m 0.1 Second radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 463110 |  | 463110 appears in the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 4788220 |  | 4788220 appears in the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Lower -> Upper button. |  | In the upper half of the interface, the Longitude field displays following value: 75 27 15.9W and the Latitude field displays the following value: 43 14 45.2N |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 0.1m 0.01 Second |  | The 0.1m 0.01 Second radio button is selected. A message box appears stating: Warning: Output accuracy is limited to ~1M |  |  |  |  |
|  |  | Left Click on the OK button in the warning box. |  | The warning message box closes. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 463100.8 |  | 463100.8 appears in the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 4788199.3 |  | 4788199.3 appears in the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Lower -> Upper button. |  | In the upper half of the interface, the Longitude field displays following value: 75 27 16.27W and the Latitude field displays the following value: 43 14 44.49N |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 0.01m 0.001 Second |  | The 0.01m 0.001 Second radio button is selected. A message box appears stating: Warning: Output accuracy is limited to ~1M |  |  |  |  |
|  |  | Left Click on the OK button in the warning box. |  | The warning message box closes. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 463100.66 |  | 463100.66 appears in the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 4788199.24 |  | 4788199.24 appears in the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Lower -> Upper button. |  | In the upper half of the interface, the Longitude field displays following value: 75 27 16.276W and the Latitude field displays the following value: 43 14 44.488N |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 0.001m 0.0001 Second |  | The 0.001m 0.0001 Second radio button is selected. A message box appears stating: Warning: Output accuracy is limited to ~1M |  |  |  |  |
|  |  | Left Click on the OK button in the warning box. |  | The warning message box closes. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 463100.664 |  | 463100.664 appears in the Easting/X (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | In the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface, enter: 4788199.240 |  | 4788199.240 appears in the Northing/Y (m) field in the lower half of the MSP GEOTRANS 3.0 interface. |  |  |  |  |
|  |  | Left click the Convert: Lower -> Upper button. |  | In the upper half of the interface, the Longitude field displays following value: 75 27 16.2759W and the Latitude field displays the following value: 43 14 44.4880N |  | SRD696 - MSP shall allow the input of coordinates for nine different levels of precision ranging from a precision of one degree to a precision of 0.0001 second. This requirement is verified when an MSP operator demonstrates the successful input of coordinates for nine different levels of precision ranging from a precision of one degree to a precision of 0.0001 second. SRD697 - MSP shall allow the input of coordinates for nine different levels of precision ranging from a precision of 100,000 meters to a precision of 0.001 meters. This requirement is verified when an MSP operator demonstrates the successful input of coordinates for nine different levels of precision ranging from a precision of 100,000 meters to a precision of 0.001 meters. SRD699 - MSP shall allow the output of coordinates for nine different levels of precision ranging from a precision of one degree to a precision of 0.0001 second. This requirement is verified when an MSP operator demonstrates the successful output of coordinates from MSP for nine different levels of precision ranging from a precision of one degree to a precision of 0.0001 second. SRD700 - MSP shall provide the output of coordinates for nine different levels of precision ranging from a precision of 100,000 meters to a precision of 0.001 meters. This requirement is verified when an MSP operator demonstrates the successful output of coordinates from MSP for nine different levels of precision ranging from a precision of 100,000 meters to a precision of 0.001 meters. |  |  |

| THIS TABLE IS UNCLASSIFIED IN ITS ENTIREITY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Input/Action |  | Expected Result/Action |  | Data Recording Comment | P/F |  |
| CCA 1-6 Access Online Help and Version Information |  |  |  |  |  |  |  |
|  |  | Left click on the Help item in the MSP GEOTRANS 3.0 menu bar. |  | The Help options appear. |  |  |  |
|  |  | Left click the following item: Contents |  | An HTML browser opens the MSP CCS User Guide. |  |  |  |
|  |  | Left click the following item: Overview |  | The Overview page appears in the browser. | SRD712 - MSP shall provide online help with information about the coordinate conversion capability and its operation. This requirement is verified when an MSP operator demonstrates that MSP provides online Help information about the coordinate conversion application and its operation. |  |  |
|  |  | Close the browser. |  | The MSP CCS User Guide is closed. |  |  |  |
|  |  | Left click on the Help item in the MSP GEOTRANS 3.0 menu bar. |  | The Help options appear. |  |  |  |
|  |  | Left click the following item: About … |  | The About box appears displaying the version information for the MSP GEOTRANS 3.0. |  |  |  |
|  |  | Left click on the OK button. |  | The About box closes. |  |  |  |

| THIS TABLE IS UNCLASSIFIED IN ITS ENTIREITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Input/Action |  | Expected Result/Action |  | Data Recording Comment |  | P/F |  |
| CCA 1-7 Individual Coordinate Conversion Testing |  |  |  |  |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Format… item. |  | The Format window opens. |  |  |  |  |
|  |  | Under Geodetic Units, select the following radio button: Degrees/Minutes/Seconds |  | The Degrees/Minutes/Seconds radio button is selected. |  |  |  |  |
|  |  | Under Geodetic Separator, select the following radio button: DD MM SS |  | The DD MM SS radio button is selected. |  |  |  |  |
|  |  | Under Sign/Hemisphere, select the following radio button: N/S, E/W |  | The N/S, E/W radio button is selected. |  |  |  |  |
|  |  | Under Longitude Range, select the following radio button: (-180, 180) |  | The (-180, 180) radio button is selected. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 1m 0.1 Second |  | The 1m 0.1 Second radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | From the Datum list field in the upper half of the interface, select: WGE: World Geodetic System 1984 |  | WGE: World Geodetic System 1984 appears in the Datum field in the upper half of the screen. |  |  |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Geodetic |  | Geodetic appears in the coordinate system field. Beneath this title, an additional field should appear with the following value: Height: Ellipsoid Height |  |  |  |  |
|  |  | From the Datum list field in the lower of the interface, select: WGE: World Geodetic System 1984 |  | WGE: World Geodetic System 1984 appears in the Datum field in the lower half of the screen. |  |  |  |  |
|  |  | From the coordinate system list field in the lower half of the interface, select: Geocentric |  | Geocentric appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: X (m): 0 Y (m): 0 Z (m): 0 |  |  |  |  |
|  |  | From the height list field in the upper half of the interface, select: Ellipsoid Height |  | Ellipsoid Height appears in the height field. |  |  |  |  |
|  |  | In the Longitude field in the upper half of the interface, enter: 180 0 0.0E |  | 180 0 0.0E appears in the Longitude field. |  |  |  |  |
|  |  | In the Latitude field in the upper half of the interface, enter: 0 0 0.0N |  | 0 0 0.0N appears in the Latitude field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The X, Y, and Z fields in the lower half of the interface will respectively display: -6378137 0 0 |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The longitude and latitude fields in the upper half of the interface will respectively display: 180 0 0.0E 0 0 0.0N |  | SRD527 - MSP shall provide coordinate conversions to and from Geodetic coordinate systems and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Geodetic coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the lower half of the interface, select: Geodetic |  | Geodetic appears in the coordinate system field. Beneath this title, an additional field should appear with the following value: Height: Ellipsoid Height |  |  |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Geocentric |  | Geocentric appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: X (m): 0 Y (m): 0 Z (m): 0 |  |  |  |  |
|  |  | In the Z field enter: 6356752 |  | 6356752 appears in the Z field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 0 0 0.0E 90 0 0.0N |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The X, Y, and Z fields in the upper half of the interface will respectively display: 0 0 6356752 |  | SRD528 - MSP shall provide coordinate conversions to and from Geocentric coordinate systems and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Geocentric coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Local Cartesian |  | Local Cartesian appears in the coordinate system field. Beneath this field, additional fields should appear with the following values: Origin Longitude: 0 0 0.0E Origin Latitude: 0 0 0.0N Origin Height (m): 0 Orientation: 0 0 0.0E X (m): 0 Y (m): 0 Z (m) : 0 |  |  |  |  |
|  |  | In the X field in the upper half of the interface, enter: -4364136 |  | -4364136 appears in the X field |  |  |  |  |
|  |  | In the Y field in the upper half of the interface, enter: 4487843 |  | 4487843 appears in the Y field |  |  |  |  |
|  |  | In the Z field in the upper half of the interface, enter: -5208770 |  | -5208770 appears in the z field |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The longitude, latitude, and height fields in the lower half of the interface will respectively display: 75 0 0.0W 45 0 0.0N 700 |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The X, Y, and Z fields in the upper half of the interface will respectively display: -4364136 4487843 -5208770 |  | SRD529 - MSP shall provide coordinate conversions to and from Local Cartesian coordinate systems and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Local Cartesian coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Albers Equal Area Conic |  | Albers Equal Area Conic appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E Origin Latitude: 45 0 0.0N Std. Parallel 1: 40 0 0.0N Std. Parallel 2: 50 0 0.0N False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing/Y (m): 0 |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 10926833 |  | 10926833 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: -1895253 |  | -1895253 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The longitude and latitude fields in the upper half of the interface will respectively display: 75 0 0.0E 35 0 0.0S |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the lower half of the interface will respectively display: 10926833 -1895253 |  | SRD530 - MSP shall provide coordinate conversions to and from Albers Equal Area Conic map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Albers Equal Area Conic coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Bonne |  | Bonne appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E Origin Latitude: 45 0 0.0N False Easting (m): 0 False Northing (m): 0 Easting (m)/X: 0 Northing (m)/Y: 0 |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 6618867 |  | 6618867 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: -7348103 |  | -7348103 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 0 0.0E 35 0 0.0S |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 6618867 -7348103 |  | SRD531 - MSP shall provide coordinate conversions to and from Bonne map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Bonne coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Cassini |  | Cassini appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E Origin Latitude: 0 0 0.0N False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Central Meridian field in the upper half of the interface, enter: 75 0 0.0W |  | 75 0 0.0W appears in the Central Meridian field. |  |  |  |  |
|  |  | In the Origin Latitude field in the upper half of the interface, enter: 40 0 0.0N |  | 40 0 0.0N appears in the Origin Latitude field. |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 163067 |  | 163067 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: 335132 |  | 335132 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 73 0 0.0W 43 0 0.0N |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 163067 335132 |  | SRD532 - MSP shall provide coordinate conversions to and from Cassini map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Cassini coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Cylindrical Equal Area |  | Cylindrical Equal Area appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E Origin Latitude: 0 0 0.0N False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Central Meridian field in the upper half of the interface, enter: 75 0 0.0W |  | 75 0 0.0W appears in the Central Meridian field. |  |  |  |  |
|  |  | In the Origin Latitude field in the upper half of the interface, enter: 5 0 0.0N |  | 5 0 0.0N appears in the Origin Latitude field. |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: -332696 |  | -332696 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: 554284 |  | 554284 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 78 0 0.0W 5 0 0.0N |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: -332696 554284 |  | SRD533 - MSP shall provide coordinate conversions to and from Cylindrical Equal Area map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Cylindrical Equal Area coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Lambert Conformal Conic (1 parallel) |  | Lambert Conformal Conic (1 Standard Parallel) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E Origin LatitudeL 45 0 0.0N Scale Factor: 1.00000 False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Origin Latitude field in the upper half of the interface, enter: 60 0 0.0N |  | 60 0 0.0N appears in the Origin Latitude field. |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 5934382 |  | 5934382 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: 918384 |  | 918384 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 0 0.0E 35 0 0.0N |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 5934382 918384 |  | SRD534 - MSP shall provide coordinate conversions to and from Lambert Conformal Conic map projections with one standard parallel and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Lambert Conformal Conic, parallel1 coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Lambert Conformal Conic (2 parallel) |  | Lambert Conformal Conic (2 parallel) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E Origin Latitude: 45 0 0.0N Std. Parallel 1: 40 0 0.0N Std. Parallel 2: 50 0 0.0N False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 5972201 |  | 5972201 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: 1872576 |  | 1872576 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 0 0.0E 35 0 0.0N |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 5972201 1872576 |  | SRD535 - MSP shall provide coordinate conversions to and from Lambert Conformal Conic map projections with two standard parallels and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Lambert Conformal Conic, parallel2 coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Mercator (Standard Parallel) |  | Mercator (Standard Parallel) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E Standard Parallel: 0 0 0.0N Scale Factor: 1.00000 False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: -8348962 |  | -8348962 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: 4139373 |  | 4139373 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 0 0.0W 35 0 0.0N |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: -8348962 4139373 |  | SRD536 - MSP shall provide coordinate conversions to and from Mercator map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Mercator coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Ney’s (Modified Lambert Conformal Conic) |  | Ney’s (Modified Lambert Conformal Conic) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E Origin Latitude: 80 0 0.0N Std. Parallel 1: 71 Std. Parallel 2: 89 59 58.0 False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 6234647 |  | 6234647 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: -599765 |  | -599765 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 0 0.0E 35 0 0.0N |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 6234647 -599765 |  | SRD537 - MSP shall provide coordinate conversions to and from Ney's map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. Note 2: Ney's projection is a version of the Lambert Conformal Conic projection that is used for submarine navigation near the North Pole. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Ney’s coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Polar Stereographic (Standard Parallel) |  | Polar Stereographic (standard Parallel) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E Standard Parallel: 90 0 0.0N False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Central Meridian field in the upper half of the interface, enter: 100 0 0.0W |  | 100 0 0.0W appears in the Central Meridian field. |  |  |  |  |
|  |  | In the Standard Parallel field in the upper half of the interface, enter: 71 0 0.0S |  | 71 0 0.0S appears in the Standard Parallel field. |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: -1539953 |  | -1539953 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: -560497 |  | -560497 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 150 0 0.0E 75 0 0.0S |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: -1539953 -560497 |  | SRD539 - MSP shall provide coordinate conversions to and from Polar Stereographic map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Polar Stereographic coordinates correctly to another coordinate system and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Polyconic |  | Polyconic appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E Origin Latitude: 0 0 0.0N False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Central Meridian field in the upper half of the interface, enter: 96 0 0.0W |  | 96 0 0.0W appears in the Central Meridian field. |  |  |  |  |
|  |  | In the Origin Latitude field in the upper half of the interface, enter: 30 0 0.0N |  | 30 0 0.0N appears in the Origin Latitude field. |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 1776728 |  | 1776728 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: 1319682 |  | 1319682 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 0 0.0 W 40 0 0.0N |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 1776728 1319682 |  | SRD540 - MSP shall provide coordinate conversions to and from Polyconic map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Polyconic coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Sinusoidal |  | Sinusoidal appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Central Meridian field in the lower half of the interface, enter: 90 0 0.0W |  | 90 0 0.0W appears in the Central Meridian field. |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 1075436 |  | 1075436 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: -5540847 |  | -5540847 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 0 0.0W 50 0 0.0S |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 1075436 -5540847 |  | SRD541 - MSP shall provide coordinate conversions to and from Sinusoidal map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Sinusoidal coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Transverse Cylindrical Equal Area |  | Transverse Cylindrical Equal Area appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E Origin Latitude: 0 0 0.0N Scale Factor: 1.00000 False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Central Meridian field in the upper half of the interface, enter: 75 0 0.0W |  | 75 0 0.0W appears in the Central Meridian field. |  |  |  |  |
|  |  | In the Origin Latitude field in the upper half of the interface, enter: 30 0 0.0N |  | 30 0 0.0N appears in the Origin Latitude field. |  |  |  |  |
|  |  | In the Scale Factor field in the upper half of the interface, enter: 0.99 |  | 0.99 appears in the Scale Factor field. |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: -687808 |  | -687808 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: 1128676 |  | 1128676 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 83 0 0.0W 40 0 0.0N |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: -687808 1128676 |  | SRD542 - MSP shall provide coordinate conversions to and from Transverse Cylindrical Equal Area map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Transverse Cylindrical Equal Area coordinates correctly to another coordinate system and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Transverse Mercator |  | Transverse Mercator appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E Origin Latitude: 0 0 0.0N Scale Factor: 1.00000 False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Central Meridian field in the upper half of the interface, enter: 75 0 0.0W |  | 75 0 0.0W appears in the Central Meridian field. |  |  |  |  |
|  |  | In the Scale Factor field in the upper half of the interface, enter: 0.9996 |  | 0.9996 appears in the Scale Factor field. |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 127103 |  | 127103 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: 4484335 |  | 4484335 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 73 30 0.0W 40 30 0.0N |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 127103 4484335 |  | SRD543 - MSP shall provide coordinate conversions to and from Transverse Mercator map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Transverse Mercator coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Azimuthal Equidistant (S) |  | Azimuthal Equidistant (S) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E Origin Latitude: 0 0 0.0N False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 7000598 |  | 7000598 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: -5074790 |  | -5074790 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 0 0.0E 35 0 0.0S |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 7000598 -5074790 |  | SRD544 - MSP shall provide coordinate conversions to and from Azimuth Equidistant map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Azimuth Equidistant coordinates correctly to another coordinate system and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Eckert IV (S) |  | Eckert IV (S) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Central Meridian field in the upper half of the interface, enter: 90 0 0.0W |  | 90 0 0.0W appears in the Central Meridian field. |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 1194736 |  | 1194736 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: -6064695 |  | -6064695 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 0 0.0W 50 0 0.0S |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 1194736 -6064695 |  | SRD545 - MSP shall provide coordinate conversions to and from Eckert IV map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Eckert IV coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Eckert VI (S) |  | Eckert VI (S) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Central Meridian field in the upper half of the interface, enter: 90 0 0.0W |  | 90 0 0.0W appears in the Central Meridian field. |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 1079009 |  | 1079009 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: -6097196 |  | -6097196 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 0 0.0W 50 0 0.0S |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 1079009 -6097196 |  | SRD546 - MSP shall provide coordinate conversions to and from Eckert VI map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Eckert VI coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Equidistant Cylindrical (S) |  | Equidistant Cylindrical (S) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E Standard Parallel: 0 0 0.0N False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 8339629 |  | 8339629 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: -3891827 |  | -3891827 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 0 0.0E 35 0 0.0S |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 8339629 -3891827 |  | SRD547 - MSP shall provide coordinate conversions to and from Equidistant Cylindrical map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Equidistant Cylindrical coordinates correctly to another coordinate, within tolerances, system and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Gnomonic (S) |  | Gnomonic (S) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E Origin Latitude: 0 0 0.0N False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 23776922 |  | 23776922 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: -17236086 |  | -17236086 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 0 0.0E 35 0 0.0S |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 23776922 -17236086 |  | SRD548 - MSP shall provide coordinate conversions to and from Gnomonic map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Gnomonic coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Miller Cylindrical (S) |  | Miller Cylindrical (S) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: -8339629 |  | -8339629 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: -6075629 |  | -6075629 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 0 0.0W 50 0 0.0S |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: -8339629 -6075629 |  | SRD549 - MSP shall provide coordinate conversions to and from Miller Cylindrical map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Miller Cylindrical coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Mollweide (S) |  | Mollweide (S) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Central Meridian field in the upper half of the interface, enter: 90 0 0.0W |  | 90 0 0.0W appears in the Central Meridian field. |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 1139674 |  | 1139674 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: -5866906 |  | -5866906 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 0 0.0W 50 0 0.0S |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 1139674 -5866906 |  | SRD550 - MSP shall provide coordinate conversions to and from Mollweide map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Mollweide coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Orthographic (S) |  | Orthographic (S) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E Origin Latitude: 0 0 0.0N False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Central Meridian field in the upper half of the interface, enter: 100 0 0.0W |  | 100 0 0.0W appears in the Central Meridian field. |  |  |  |  |
|  |  | In the Origin Latitude field in the upper half of the interface, enter: 40 0 0.0N |  | 40 0 0.0N appears in the Origin Latitude field. |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: -958096 |  | -958096 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: -1052434 |  | -1052434 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 110 0 0.0W 30 0 0.0N |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: -958096 -1052434 |  | SRD551 - MSP shall provide coordinate conversions to and from Orthographic map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Orthographic coordinates correctly to another coordinate system and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Stereographic (S) |  | Stereographic (S) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Appropriate parameter field: Central Meridian: 0 0 0.0E Origin Latitude: 0 0 0.0N False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 8318393 |  | 8318393 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: -6030071 |  | -6030071 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 0 0.0E 35 0 0.0S |  | SRD552 - MSP shall provide coordinate conversions to and from Stereographic map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Sterographic coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 8318393 -6030071 |  |  |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Van der Grinten |  | Van der Grinten appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Central Meridian: 0 0 0.0E False Easting (m): 0 False Northing (m): 0 Easting /X (m): 0 Northing /Y (m): 0 |  |  |  |  |
|  |  | In the Central Meridian field in the upper half of the interface, enter: 85 0 0.0W |  | 85 0 0.0W appears in the Central Meridian: 0 0 0.0E field. |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: -7616000 |  | -7616000 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: -6345990 |  | -6345990 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 160 0 0.0W 50 0 0.0S |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: -7616000 -6345990 |  | SRD553 - MSP shall provide coordinate conversions to and from Van der Grinten map projections and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Van der Grinten coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: GEOREF |  | GEOREF appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Coordinate String |  |  |  |  |
|  |  | In the Coordinate String field in the upper half of the interface, enter: GJQF4500030000 |  | GJQF4500030000 appears in the Coordinate String field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 15 0.0W 35 30 0.0N |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Coordinate String field in the upper half of the interface will display: GJQF4500030000 |  | SRD554 - MSP shall provide coordinate conversions to and from the World Geographic Reference System (GEOREF) grid and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input World Geographic Reference System coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in string format must match exactly. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Universal Polar Stereographic (UPS) |  | Universal Polar Stereographic (UPS) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Hemisphere: N Easting/X (m): 2000000 Northing/Y (m): 2000000 |  |  |  |  |
|  |  | Select the south hemisphere radio button in the upper half of the interface. |  | The south hemisphere option is selected. |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 2222979 |  | 2222979 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: 1797475 |  | 1797475 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 132 14 52.5E 87 17 14.4S |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 2222979 1797475 |  | SRD555 - MSP shall provide coordinate conversions to and from the Universal Polar Stereographic (UPS) grid and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Universal Polar Stereographic (UPS) coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Universal Transverse Mercator (UTM) |  | Universal Transverse Mercator (UTM) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Overide: Unchecked Zone: 0 Hemisphere: N Easting/X (m): 500000 Northing/Y (m): 0 |  |  |  |  |
|  |  | In the Zone field in the upper half of the interface, enter: 18 |  | 18 appears in the Zone field |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 500000 |  | 500000 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: 3873043 |  | 3873043 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 0 0.0W 35 0 0.0N |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 500000 3873043 |  | SRD556 - MSP shall provide coordinate conversions to and from the Universal Transverse Mercator (UTM) grid and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Universal Transverse Mercator (UTM) coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in meters must match to within 1 meter. UTM zone designator must match exactly. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Military Grid Reference System (MGRS) |  | Military Grid Reference System (MGRS) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Coordinate String: 31NEA0000000000 |  |  |  |  |
|  |  | In the Coordinate String field in the upper half of the interface, enter: 18SWD0000073043 |  | 18SWD0000073043 appears in the Coordinate String field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will display: 75 0 0.0W 35 0 0.0N |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Coordinate String field will display: 18SWD0000073043 |  | SRD557 - MSP shall provide coordinate conversions to and from the Military Grid Reference System (MGRS) and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Military Grid Reference System (MGRS) coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. For coordinates in string format: Zone designator substring must match exactly. 100,000 meter grid square substring must match exactly. Easting and Northing substrings must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: British National Grid (BNG) |  | British National Grid (BNG) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Coordinate String: SV 0000000000 In addition, the Convert: Upper -> Lower and the Convert: Lower -> Upper buttons will turn red because BNG only works with the Airy Ellipsoid. This will be addressed in the next step. |  |  |  |  |
|  |  | From the Datum list field in the upper of the interface, select: OGB-A: ORDNANCE GB 1936, ENGLAND |  | OGB-A: ORDNANCE GB 1936, ENGLAND appears in the Datum field in the upper half of the screen. In the ellipsoid field in the upper half of the interface the following appears: AA: Airy 1830 Additionally, the Convert: Upper -> Lower and the Convert: Lower -> Upper buttons will turn green. |  |  |  |  |
|  |  | In the Coordinate String field in the upper half of the interface, enter: TN 4324945318 |  | TN 4324945318 appears in the Coordinate String field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will display: 3 0 0.0E 52 0 0.0N |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Coordinate String field in the upper half of the interface will respectively display: TN 4324945318 |  | SRD558 - MSP shall provide coordinate conversions to and from the British National Grid (BNG) and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input British National Grid (BNG) coordinates correctly to another coordinate system and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. For coordinates in string format: 500,000 meter grid square substring must match exactly. 100,000 meter grid square substring must match exactly. Easting and Northing substrings must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: New Zealand Map Grid |  | New Zealand Map Grid appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Easting /X (m): 2510000 Northing /Y (m): 6023150 In addition, the Convert: Upper -> Lower and the Convert: Lower -> Upper buttons will turn red because New Zealand Map Grid only works with the International 1924 ellipsoid. This will be addressed in the next step. |  |  |  |  |
|  |  | From the Datum list field in the upper of the interface, select: GEO: GEODETIC DATUM 1949, NZ |  | GEO: GEODETIC DATUM 1949, NZ appears in the Datum field in the upper half of the screen. In the ellipsoid field in the upper half of the interface the following appears: IN: International 1924 Additionally, the Convert: Upper -> Lower and the Convert: Lower -> Upper buttons will turn green. |  |  |  |  |
|  |  | In the Easting/X field in the upper half of the interface, enter: 2693454 |  | 2693454 appears in the Easting/X field. |  |  |  |  |
|  |  | In the Northing/Y field in the upper half of the interface, enter: 6686794 |  | 6686794 appears in the Northing/Y field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 175 0 0.0E 35 0 0.0S |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Easting/X and Northing/Y fields in the upper half of the interface will respectively display: 2693454 6686794 |  | SRD559 - MSP shall provide coordinate conversions to and from the New Zealand Map Grid (NZMG) and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input New Zealand Map Grid (NZMG) coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in string format must match exactly. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: United States National Grid (USNG) |  | United States National Grid (USNG) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Coordinate String: 31NEA0000000000 |  |  |  |  |
|  |  | From the Datum list field in the upper of the interface, select: WGE: World Geodetic System 1984 |  | WGE: World Geodetic System 1984 appears in the Datum field in the lower half of the screen. In the ellipsoid field in the lower half of the interface the following appears: WE: WGS 1984 |  |  |  |  |
|  |  | In the Coordinate String field in the upper half of the interface, enter: 18SWD0000073043 |  | 18SWD0000073043 appears in the Coordinate String field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the upper half of the interface will respectively display: 75 0 0.0W 35 0 0.0N |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Coordinate String field in the upper half of the interface will display: 18SWD0000073043 |  | SRD561 - MSP shall provide coordinate conversions to and from the United States National Grid (USNG) and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input United States National Grid (USNG) coordinates correctly to another coordinate,within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. For coordinates in string format: Zone designator substring must match exactly. 100,000 meter grid square substring must match exactly. Easting and Northing substrings must match to within 1 meter. |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: Global Area Reference System (GARS) |  | Global Area Reference System (GARS) appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Coordinate string: 361HN37 |  |  |  |  |
|  |  | In the Coordinate String field in the upper half of the interface, enter: 210LM47 |  | 210LM47 appears in the Coordinate String field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 75 12 30.0W 35 32 30.0N |  | SRD562 - MSP shall provide coordinate conversions to and from the Global Area Reference System (GARS) and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input Global Area Reference System (GARS) coordinates correctly to another coordinate system, within tolerances, and convert those coordinates back showing the same value as the input coordinate, within tolerances. |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Coordinate String field in the upper half of the interface will display: 210LM47 |  |  |  |  |
|  |  | From the coordinate system list field in the upper half of the interface, select: F-16 Grid Reference System |  | F-16 Grid Reference System appears in the coordinate system field. Beneath this title, additional fields should appear with the following values: Coordinate String: 31NEA00 |  |  |  |  |
|  |  | Left click on the Options item in the MSP GEOTRANS 3.0 menu bar. |  | The Option selections appear. |  |  |  |  |
|  |  | Left click on the Precision tab. |  | The Precision options appear. |  |  |  |  |
|  |  | Under Precision, select the following radio button: 100,000 m 1 Degree |  | The 100,000 m 1 Degree radio button is selected. |  |  |  |  |
|  |  | Left Click on the OK button in the Format window. |  | The Format window closes. |  |  |  |  |
|  |  | In the Coordinate String field in the upper half of the interface, enter: 60HUG00 |  | 60HUG00 appears in the Coordinate String field. |  |  |  |  |
|  |  | Left click on the Convert: Upper -> Lower button. |  | The Longitude and Latitude fields in the lower half of the interface will respectively display: 175E 35S |  |  |  |  |
|  |  | Left click on the Convert: Lower -> Upper button. |  | The Coordinate String field in the lower half of the interface will display: 60HUG00 |  | SRD560 - MSP shall provide coordinate conversions to and from the F-16 Grid Reference System and the other coordinate systems listed in Table 3.2-4 "Projections and Grid Reference Systems" in the MSP SRD. Note 3: The F-16 Grid Reference System is a variant of MGRS that is intended to be compatible with the navigation system of the F-16 fighter aircraft. It appends two trailing zeros to the designation of an MGRS 100,000m square. This requirement is verified when an MSP operator demonstrates the MSP capability to convert input F-16 Grid Reference System coordinates correctly to another coordinate,, within tolerances, system and convert those coordinates back showing the same value as the input coordinate, within tolerances. MSP CCA individual coordinate testing: Converts coordinates from one system to another. Coordinates in decimal degrees must match to within 0.00001º. Coordinates in string format must match exactly. |  |  |